ON THE REACTIONS IN ONE DEGREE OF FREEDOM

MULTIBODY SISTEMS

Sorin Vlase, Arina Modrea, Violeta Munteanu, Apopei Ramona

„Transilvania” University of Brasov, Romania

Abstract  In the paper is presented a multibody system with one degree of freedom, the aim being to compute the reactions of a such system. First, a kinematical analysis presents the liaisons between the generalized coordinates. These results are used in the dynamical analysis of the whole mechanism. Finally, the motion equations are presented. A natural method to eliminate the liaison forces is presented, method that replaces the well known method of the inversion of the coefficients matrix with a simple matrix multiplication that lightens significantly the numerical approach of the determination of reactions that appear in one freedom degree mechanisms.

Keywords: Multibody systems, liaison forces, virtual mechanical work

# 1. INTRODUCTION

In the following, a natural method to eliminate the constraint forces is presented, method that replaces the inversion of the coefficients matrix with a simple matrix multiplication, which lightens significantly the numerical approach of such problems. After this operation the determination of exterior reactions is an elementary operation.

# 2. KINEMATICS

We summarize the known results to express the kinematical conditions. Since, to write the motion equations is convenient to use the coordinates systems situated in the center of mass, we will compute the velocity and acceleration of the center of mass for each element. We will note with x the vector which contains on the first three positions the linear velocity components of the center of mass and on the following three positions, the components of the angular velocity vector of the element i. We will note with x the vector which contains, in order, the linear acceleration and angular acceleration components of the center of mass of element i. If s is the mobility degree of the system imposed by constraints, then the vectorsx, i=1, 2, .,n, where n is the number of bodies in constraint, can be linear expressed as a function of s derivatives qj, i=1,2,.. .,s. We noted with qj the independent coordinates of the system. So, it could be writing:

x=x1x2⋮xn=[A]q1q2⋮qs=[A]q				      (1)

In the following, it will be note:

q2=[ q12 q1q2 q1q3  …    {qs2 ]T					(2)

The vectors xi i =1, 2, .,n can be expressed by a linear function of qj and a quadratic function of qj, j=1,2,.. .,s. We have:

x=[A]q+[B]q2						(3)

relation obtained through the derivative of expression (1).

# 3. MOTION EQUATIONS FOR MULTIBODY SYSTEMS

We note with mi the mass of the element i, with [J]i the matrix of inertia moments computed in the center of mass, Qiext the equivalent force-couple system of the exterior forces reduced in the center of mass and with Qileg the equivalent force-couple system of the constraint forces reduced in the center of mass. The motion equations for the element i can be written in the center of mass of the body:

[M]ixi+M'ixi=Qiext+Qileg

For the entire system we can write concise:

[M]x+M'x=Qext+Qleg					(4)

If we take equation (3) into account we can write down:

[M][A]q+([M][B]+[M'][A])q2=Qext+Qleg			(5)

which is a system of differential equations in independent coordinates q. In the equations appears the equivalent force-couple system of the constraint forces also with components in relation to each local coordinates system, disposed in the center of mass. In general, if n is the number of bodies, s the mobility degree, appear 6n – s constraint forces. If we note with Fi, i=1,2,.. ., 6n – s  the constraint forces, we can write down:

Qleg=[T]F					  	(6)

where matrix [T] depends on the system geometry. If the number of constraint forces is grater than 6n – s we deal with a non-determined static structure, for the determination of the constraint forces, the elastic properties of the elements must be used.

# 4. WORK OF THE LIAISON FORCES

Let us consider vCi the center of mass velocity of the element i of the system; Ri a constraint force and A a point in which Ri acts (fig. 1). The velocity of point A will be:

vAi=vCi+ω irAi				      (7)

where ω  i  is the angular velocity operator of element i. The elementary mechanical work of the force Ri at a displacement compatible with the constraints is:

dLi=RiTvAidt=RiT(vCi+[ω]i rAi )dt=

=(vCiTRi+ωiT[rA]iRi)dt=xiTRi[rA]iRi			(8)

But expression (8) represents the equivalent force-couple system of the force Ri in point C. If we take into account all constraint forces and compute the total elementary mechanical work of the constraint forces that acts upon the element, we obtain:

dLie=xiTRMA=xiTQileg 				(9)

where Qileg is the equivalent force-couple system of the constraint forces in the center of mass C. For the whole system we can write down:

dL=dL1e+dL2e+…+dLne=x1TQ1leg+…+xnTQnleg=xTQleg.

But the mechanical work of the constraint forces is zero, for the entire system we obtain:

xTQleg=0					          (10)

Fig. 1. A rigid body from the system, subjected to constraints

# 5. ELIMINATION OF CONSTRAINT FORCES

If we take into account the kinematics conditions (1), the expression (10) can be written:

qTATQleg=0					(11)

but the coordinates  q  are independent, so that we obtain:

ATQleg=0   					(12)

This important result that can be proved in some general conditions will be used to eliminate the constraint forces from (5). So, pre-multiplying these relations with [A]T, we obtain:

[A]T[M][A]q+[A]T([M][B]+[M'][A])q2=[A]TQleg+[A]TQext	(13)

and using (12), we can write down:

[A]T[M][A]q+[A]T([M][B]+[M'][A])q2=[A]TQext			(14)

which represents a system of s second degree equations from which the constraint forces are missing. If we integrate this system, the constraint forces can be easily obtained. So, essentially, the elimination of constraint forces from the equations, reduces itself to the system multiplication with the matrix [A]T.

# 6. AN EXAMPLE

We propose to determine the motion equations for a crank – connecting rod mechanism.

Fig. 2. The crank – connecting rod mechanism

Are given: r – the connecting rod radius; l – the crank length; a – the position of the connecting rod center of mass against point A; b – the position of the crank center of mass against point B.

Kinematical conditions (holonomic constraints):

xC1=acosα ; yC1=asinα ; xC2=rcosα+bcosβ ;yC2=rsinα−bsinβ ; xC=rcosα+lcosβ ; rsinα=lsinβ ;		(15)

If the last relation is derived, we obtain:

rαcosα=lβsinβ

or, if we take into account that:

α=ω1 ; β=−ω2 ; ω1=ε1 ; ω2=ε2

it results:

rω1cosα=−lω2cosβ

or:

ω2=−rcosαlcosβω1=tω1  ;  with  t=−rcosαlcosβ

and:

rε1cosα−rω12sinα=−lε2cosβ−lω22sinβ

where:

ε2=−rcosαlcosβε1+rsinαlcosβ−t2sinβcosβω12=tε1+uω12 with u=rsinαlcosβ−t2sinβcosβ

Therefore, the first relations, derived, will give:

xC1yC1ω1xC2yC2ω2xC=−aω1sinα aω1cosα1−rsinα+btsinβrcosα+btcosβt−rsinα+ltsinβω1=A1ω1			(16)

If we derive once more the kinematics conditions we obtain:

xC1yC1ε1xC2yC2ε2xC=−asinα acosα1−rsinα+btsinβrcosα+btcosβt−rsinα+ltsinβε1+−acosα−asinα0−rcosα−bt2sinβ+businβ−rsinα+bt2sinβ+bucosβu−rsinα+lt2sinβ+lucosβω12	(17)

or in a compact writing:

a=A1ε1+A2ω12					(17’)

Motion equations:

A possibility to obtain the motion equations is to apply the basic theorems for a rigid body. We consider the mechanism as being composed of three rigid bodies.

For the AB rod we obtain two equations from the linear momentum theorem and one considering the kinetic moment theorem:

m1xC1=XA+XBm1yC1=YA+YB

JC1ε1=Mm+XAr2sinα−YAr2cosα−XBr2sinα+YBr2cosα

Fro the BC rod we obtain:

m2xC2=−XB+XCm2yC2=−YB+YC

JC2ε2=XCl2sinβ+YCl2cosβ+XBl2sinβ+YBl2cosβ

Point C will have a rectilinear motion, so we can write:

m3xC=Fr−XC.

If we write all the grouped equations, we obtain:

m10000000m10000000JC10000000m20000000m20000000JC20000000m3xC1yC1ε1xC2yC2ε2xC=XA+XBYA+YBMm+XAasinα−YAacosα−XB(r−a)sinα+YB(r−a)cosα−XB+XC−YB+YCXC(l−b)sinβ+YC(l−b)cosβ+XBbsinβ+YBbcosβFr−XC(18)

Fig. 3. The system separation in component parts

or, if we take into account the kinematics conditions:

m10000000m10000000JC10000000m20000000m20000000JC20000000m3(−asinα acosα1−rsinα+btsinβrcosα+btcosβt−rsinα+ltsinβε1+−acosα−asinα0−rcosα−bt2sinβ+businβ−rsinα+bt2sinβ+bucosβu−rsinα+lt2sinβ+lucosβω12)=

=00Mm000Fr+101000010100asinα−acosα−(r−a)sinα(r−a)cosα0000−1010000−10100bsinβbcosβ(l−b)sinβ(l−b)cosβ0000−10XAYAXBYBXCYC           (19)

If we consider the previous notations we can write under grouped form:

mA1ε1+A2ω12=Qext+Qleg=Qext+NR			(19’)

where the notations are obvious.

Mechanical work of the constraint forces:

The mechanical work of the constraint forces can be written as:

dL=δ ΔTQleg=ΔTQlegdt=qTA1TQlegdt=ω1A1Qlegdt

But we have the relation:

A1TQleg=0 so   dL=0 .

The vector Qleg represents the equivalent force-couple system of the generalized constraint forces according to the considered generalized coordinates. It results the motion equation:

J(α)α+J'(α)α2=M(α)			                       (20)

where the notations has been used:

J(α)=A1TmA1=[m1a2+JC1+m2r2+m2b2t2++2m2rbtcos(α+β)+JC2t2+m3(−rsinα+ltsinβ)2] ;

J'(α)=A1TmA2=

=[m2(rbt2cos(α−β)+rbucos(α+β)−rbtsin(α+β)−b2t3cos2β++b2tu+JC2tu+m3(−rsinα+ltsinβ)(−rsinα+lt2sinβ+lucosβ)]

M(α)=Mm+Fr−rsinα+ltsinβ ,

We obtain the motion equation under the form:

ddtJ(α)α22=M(α)α				           (21)

It results:

J(α)α22=M(α)dα.				           (22)

α2=2J(α)0αM(θ)dθ;				          (23)

and:				α=1J(α)M(α)−J'(α)2J(α)0αM(θ)dθ			          (24)

# REFERENCES

[1]. Iacob,C., Mecanica teoretica,  Edit.  did. si  ped., Bucuresti, 1980

[2]. Paul,B., Analytical Dynamics of Mechanisms - A Computer Oriented Overview, Mechanism and   Machine Theory, 10, p.481-507, 1975

[3]. Vlase, S., A Method of Eliminating Lagrangean  Multipliers from the Equation of Motion Of  Interconnected Mechanical Systems, Journal of Applied Mechanics, vol.54,p235-237,1987.

[4]. Lanczos, C., The Variational Principles of Mechanics, University of Toronto, Toronto, Canada, 1966