STABILITY OF THE BENDING VIBRATIONS OF A NONCIRCULAR CROSS-SECTION SHAFT

Cr. Dragomirescu1, A. Craifaleanu2, V. Ceauşu3

1 “Politehnica” University of Bucharest, Romania,  dragom@cat.mec.pub.ro

2 Politehnica” University of Bucharest, Romania,  craifaleanu@cat.mec.pub.ro

3 Politehnica” University of Bucharest, Romania,  ceausu@cat.mec.pub.ro

Abstract:  The paper studies the motion of a system consisting in a shaft with noncircular cross-section and a rotor. The conditions are determined, under which the bending vibrations are unstable, for different values of the parameters. The system has two degrees-of-freedom and is autonomous. The stabilization of the system by using feed-back control circuits is considered.

Keywords:  bending vibrations, feed-back control, noncircular cross-section, shaft.

1. PESENTATION OF THE MODEL

The paper studies a noncircular cross-section shaft, of length L and negligible mass, with a rotor of mass M, fixed at its middle. The system has a rotation motion, with the angular velocity =constant (Fig. 1).

| Figure 1: Rotating shaft | Figure 2: Cross-section displacement |
| --- | --- |

In order to determine the differential equations of the bending vibrations of the shaft, two coordinate systems are used (Fig. 2):

- the fixed one X0Y0Z0;

- the movable one, XYZ, rotating together with the shaft, chosen so that the axes X and Y are parallel to the principal axes of inertia of the rotor X’Y’.

The mass centre of the rotor coincides with the mass centre of the shaft and the gyroscopic effect is neglected.

The position of the mass center, with respect to the two coordinate systems, is defined by the vector

r=x0i0+y0j0=xi+yj.								()

In the kinematics of the relative motion, the accelerations can be composed according to the well-known formula [3]

a=ar+at+ac,										()

where the following notations were used:

- the absolute acceleration

a=r,											()

the relative acceleration

ar=∂2r∂t2=xi+yj,										()

the transport acceleration

at=aO+ε×r+Ω×Ω×r=−Ω2xi−Ω2yj							()

and the Coriolis’ acceleration.

ac=2Ω×vr=−2Ωyi+2Ωxj.									()

The formulae above have been written considering the relations:

aO=0,											()

Ω=Ωk=const,										()

ε=Ω=0.											()

The fundamental equation of the dynamics will be applied,

ma=F,											()

considering that the force F represents the action of the weight

Mg=−MgisinΩt+jcosΩt									()

and of the elastic force

Fe=−k1xi−k2j,										()

where the rigidities are determined by formulae [1]

k1=48EIyL3,     k2=48EIxL3.									()

By introducing expressions ()-() and ()-() in (), the differential equations of the bending vibrations of the shaft are obtained:

Mx−2MΩy−MΩ2x+k1x+MgsinΩt=0							()

Equations () show that the system is subject to a harmonic perturbation.

2. THE STABILITY OF THE FREE VIBRATIONS

The free vibrations are obtained when the terms containing the weight are neglected.

By introducing the notations

ω12=k1M=48EIyML3,ω22=k2M=48EIxML3ω2ω1=IxIy=α,Ωω1=β ,							()

as well as the non-dimensional time

τ=Ωt=βω1t,										()

equations () become

d2xdτ2−2dydτ+1β2−1x=0									()

This system has solutions of the form

x=C'eλτ, y=C''eλτ,									()

where C’ and C” are constants.

The algebraic system that results by substituting relations () in (),

C'λ2−2C''λ+1β2−1C'=0								()

must admit nontrivial solutions, which is equivalent to the condition

λ2+1β2−1−2λ2λλ2+α2β2−1=0.									()

By expanding the determinant, the characteristic equation is obtained, whose roots are

λ1,2,3,4=±−1−12β21+α2±1+12β21+α22−1−1β2 1−α2β2.				()

The general solution of the system () is a linear combination of the fundamental solutions () and it is bounded only if the roots () are purely imaginary, which is equivalent to

1−1β2 1−α2β2>0,										()

since

1+12β21+α2>0 and 1+12β21+α22>1−1β2 1−α2β2					()

for any values of parameters  and .

Inequality () is fulfilled only if

β>1 and α<β										()

or

β<1 and α>β.										()

From the conditions above, the stability and instability domains shown in Figure 3 result.

Figure 3:  Stability and instability domains

3. THE DESIGN OF A DYNAMIC SYSTEM WITH PRESCRIBED EIGENVALUES (BY USING FEED-BACK CONTROL CIRCUITS)

By using the notations

x1=x, x2=dxdτ, x3=y, x4=dydτ, δ=1-1β2, ν=1-α2β2					()

and by introducing the perturbation u(), system () becomes

dx1dτ=x2dx2dτ=2x4+δx1dx3dτ=x4.									()

The matrix of the system is

A=0100δ00200010−2ν0										()

and its eigenvalues are the roots of the characteristic equation

−λ100δ−λ0200−λ10−2ν−λ=0									()

or, equivalent,

λ4+λ2(4−δ−ν)+δν=0.									()

It follows that the eigenvalues are

λ1,2,3,4=±124−δ−ν±4−δ−ν2−4δν.							()

If matrix [A] has real negative eigenvalues or complex ones, with negative real parts (the equilibrium configuration x = 0 is asymptotically stable), the system will behave satisfactorily, oscillating with damping about the equilibrium configuration and tending towards this configuration when the perturbation stops. For small moduli of the real part, the time interval needed for the system the reach this configuration is relatively high.

If matrix [A] has real positive eigenvalues or complex ones, with positive real parts, the system is unstable, i.e. it evolves far from the equilibrium configuration, even for small perturbations u().

If a small number of pairs of purely imaginary roots exists, under the action of the perturbation u(), the system neither tends towards the equilibrium configuration, nor evolves to far from it.

As an example, numerical values corresponding to the instability domain in Figure 3 have been chosen for the parameters of the system (β=2, α=3, β>1, α>β):

δ=1−1β2=0.75, ν=1-α2β2=−1.25.								()

It follows that

λ1=−0.45, λ2=0.45, λ3=−2.17i, λ4=2.17i,						()

i.e. the system is unstable because of the real positive value λ2.

Figure 4:  Block diagram of the uncontrolled system

In order to get the block scheme of the unstablized system (Fig. 4), the operator D will be used, so that equations () become

Dx1=x2, Dx2=2x4+δx1, Dx3=x4, Dx4=−2x2+νx3+uτ,				()

hence

x1=1Dx2, x2=2DD−δDx3, x3=1Dx4, x4=uτD+4D−δD−νD.					()

Figure 5:  Block diagram of the feed-back controlled system

By considering that rootsρ1, ρ2, ρ3, ρ4 are given, negative or one negative and two complex conjugate one, with negative real part (large enough in absolute value) and by replacing

uτ=g1x1+g2x2+g3x3+g4x4+uτ							()

in (), the system

dx1dτ=x2dx2dτ=2x4+δx1dx3dτ=x4						()

is obtained, whose matrix is

A=0100δ0020001g1g2−2g3+νg4.								()

Its characteristic equation is

−ρ100δ−ρ0200−ρ1g1g2−2g3+νg4−ρ=0								()

or

ρ4−g4ρ3+ρ2(−δ−ν−g3−2g2+4)+ρδg4−2g1+δ⋅ν+δg3=0				()

and, by using Viète’s formulae,

g4=ρ1+ρ2+ρ3+ρ4−δ−g3−ν−2g2+4=ρ1ρ2+ρ1ρ3+ρ1ρ4+ρ2ρ3+ρ2ρ4+ρ3ρ4−δg4+2g1=ρ1ρ2ρ3+ρ1ρ2ρ4+ρ1ρ3ρ4+ρ2ρ3ρ4				()

the expressions for g1, g2, g3, g4 can be determined:

g1=12ρ1ρ2ρ3+ρ1ρ2ρ4+ρ1ρ3ρ4+ρ2ρ3ρ4+δρ1+ρ2+ρ3+ρ4g2=12−δ−ν+4−1δρ1ρ2ρ3ρ4−δν−ρ1ρ2+ρ1ρ3+ρ1ρ4+ρ2ρ3+ρ2ρ4+ρ3ρ4g3=1δρ1ρ2ρ3ρ4−δν		()

The block scheme of the stabilized system is shown in Figure 5.

REFERENCES

- Deciu E., Dragomirescu Cr.: Maschinendynamik, Ed. Printech, Bucureşti, 2001.

- Holzweiβig D., Fischer St.: Maschinendynamic Schwingungslehre, VEB Fachbuchverlag, Leipzig, 1987.

- Voinea R., Voiculescu D., Ceauşu V.: Mecanica, Editura Didactică şi Pedagogică, Bucureşti, 1983.

- Voinea R., Stroe I.: Introducere în teoria sistemelor dinamice, Ed. Academiei Române, Bucureşti, 2000.