STRUCTURAL MODELLING BY MBS METHOD APPLIED TO LINKAGES USED TO AILERON AIRCRAFT CONTROL

Marius GHITESCU

Department of Product Design  and Robotics
TRANSILVANIA University of Brasov  Bv. Eroilor 29,  Brasov Romania  Email  motanmotan34@yahoo.com

Abstract: The aim of the paper is structural  modelling of linkages used in airplane flaps control, on bases of the Multibody System method (MBS). In virtual prototyping of the aircraft , these linkages have to be modeled by a minimum number of bodies (MBS min). In the paper an appropiate algorithm is descriebed and also applied for  concrete mechanical systems. This will be the bases for  dynamic modelling of these subsystems as parts of the whole product (airplane).

Keywords: mechanical systems, multibody systems, structural modeling, kinematic analysis, aircraft, Flaps

1. INTRODUCTION

Generally control surfaces of an aircraft include : ailerons , rudder ,  elevator , tabs and flaps[6].

(see figure1). Movement of this surfaces generate roll, pitch and yaw. During the fly flaps move between  “cruise posision” and “landing position” by intermediate of mechanical transmissions.

In design process of an aircraft one of the main step is virtual prototyping.

In virtual prototyping of the aircraft, mechanical systems are considered as Multibody Systems (MBS). They have to be modelled as MBS with minimum number of bodies (MBS min) to favorise obtaining real time simulation of aircraft.

The aim of this paper is structural and  kinematic modelling of linkages used for actioning of aileron , as multibody systems.  On bases of MBS, kinematic and dynamic models could be obtained (see Part02).

Aileron

Elevator

Rudder

Fig 1

2. THEORETICAL CONSIDERATION

According to MBS theory , a mechanical system is considered as a collection of bodies linked between them by geometrical and driving constraints[4,9].

The body is an entity which in the dynamic model will have mass and moment of inertia, and also could take over the external forces.

Motion of a rigid body in space can be described by six generalized coordinates - Sx ,  Sy ,  Sz  = translation,  φz  ,φy  ,φz = rotation (x,y,z are the orthogonal axes) Planar motion has Sx ,  Sy ,  φz

Global Reference Frame GRF is a coordinate system represented three orthogonal axes fixed in time.  This coordinate system is rigidly connected to fixed body [4,9].

Body Reference Frame BRF is a coordinate system formed by three orthogonal axes connected to each mobile body component to the MBS system[4,9].

In a linkage having “n” elements number of bodies nb is :

nb    n

(1)

Generally:

nb min    nb    n

nb min representing the minimum number of bodies for modeling a concrete linkage[11]

The geometrical constraint imposes restrictions in bodies relative motion. Number of restriction is

r =1 and r = 2 in the  case of planar systems (S =3), respectiv r = 1…5 in the case of three dimensional systems (S = 6).

The restrictions are imposed by joints or composite joint [4,9]. In planar systems they are: rotation R

(r =2), translation T (r =2), rotation-rotation RR (r =1), rotation –translation RT (r =1)curve –curve CC (r = 1) [4,9,11].

Driving constraints corespond to the mobility M of the system Of course M  1 .Between number of bodies nb  number of geometrical constraints (Σr) in a concrete space S , and mobility M there is the relation [11]

(2)

M = S (nb  - 1) – Σ r

The algorithm for MBS modelling with minimum number of bodies has the following steps:

- Identifying the bodies , in order:

- fixed body,

- input body (bodies),

- output body (bodies),

- bodies with more than two connection,

- bodies with applied forces,

- other bodies (if necessary).

- Identifying geometrical constraints:

- type ,

- location,

- number of restriction.

3.  STRUCTURAL  ANALYSIS OF CONCRETE LINKAGE

In the paper , one linkage used for aileron control is analized.

The bases of this linkage is wing structure , that represent for the linkage – fixed body. The output body is the ailerons. The input motion is generated by pilot from control lever by intermediate of mechanical /  hydraulic actuators.

The linkage from fig 2  is modeled as MBS min  having 6 bodies, five geometrical constraints type R and four geometrical constraints type SS. The input  body is body 2 and the output bodies are the ailerons 4 and 6. Mobility

M =1.

Figure 3 represent structural scheme at this aileron mechanism.

Fig 2

4

2

6

5

3

Fig 3

Table 1: aileron control (see fig 3)

| Body i Body j | gc | Location | Number of M = S (nb - 1) – Σ r Σ r = 29 nb = 6 si S = 6 M = 1 constraints |
| --- | --- | --- | --- |
| 1-2 | R | O2 | 5 |
| 1-3 | R | O3 | 5 |
| 1-4 | R | O4 | 5 |
| 1-5 | R | O5 | 5 |
| 1-6 | R | O6 | 5 |
| 2-3 | SS | AB | 1 |
| 2-4 | - | - | - |
| 2-5 | SS | A1B1 | 1 |
| 2-6 | - | - | - |
| 3-4 | SS | CD | 1 |
| 3-5 | - | - | - |
| 3-6 | - | - | - |
| 4-5 | - | - | - |
| 4-6 | - | - | - |
| 5-6 | SS | C1D1 | 1 |

XA−XB2+YA−YB2+ZA−ZB2=AB2

(3)

XAYA

XA(2),YA(2), ZA(2) are local coordinates (coordinates of points into BRF). O2X2Y2Z2  represente BRF for body2.

Rotation matrix is:

M21=cosϕ2−sinϕ2sinϕ2cosϕ2

(4)

Global coordinates for O2 point are:

XO2=YO2=YO2= 0

For B point:

XBYB

Where

(5)

XB(3)=ZB(3)=0

Then:

XAYA    si    XBYB

We  know:

XO2,YO2,ZO2 XO3,YO3,ZO3, XA(2),YA(2),ZA(2),XB(3),YB(3),ZB(3)

Generally rotation matrix is:

Mi=cos(Xi,X)cos(Yi,X)cos(Zi,X)cos(Xi,Y)  cos(Yi,Y)  cos(Zi,Y)

(6)

For body  “2”

M21= Mβ and  Mβ=cosβ    0    sinβ0            1      0

(7)

Result:

XAYA   si  XBYB

Where:

Mα=cosα    −sinα    0sinα       cosα      0

(8)

Result:

XA=XA(2)cosβ+ZA(2)sinβYA=YA(2)

(9)

For B point:

XBYB

(10)

Result:

XB=XO3−YB(3)sinαYB  =  YO3+YB(3)cosα

(11)

Final relation   go to:

XA(2)cosβ+ZA(2)sinβ−XO3+YB(3)sinα2+YA(2)−YO3+YB(3)cosα2++−XA(2)sinβ  +  ZA(2)cosβ−ZO32=AB2

(12)

Between bodies “2” and “5” we have restriction A1B1 type SS:

(13)

XA1−XB12+YA1−YB12+ZA1−ZB12=A1B12

Where:

XA1YA1  si  XB1YB1

(14)

And:

M21=Mβ  ,   M51=Mα

Mβ=cosβ    0    sinβ0            1      0         and       Mα=cosα    −sinα    0sinα       cosα      0

Result:

XA1=XA1(2)cosβ+ZA1(2)sinβYA1=YA1(2)      si      XB1=XO5−YB1(5)sinαYB1  =  YO5+YB1(5)cosα

(15)

Then previous relation go to:

XA1(2)cosβ+ZA1(2)sinβ−XO5+YB1(5)sinα2+YA1(2)−YO5+YB1(5)cosα2++−XA1(2)sinβ  +  ZA1(2)cosβ−ZO52=A1B12

(16)

Between bodies “3” and “4”  we have geometrical constraint CD type SS:

XC−XD2+YC−YD2+ZC−ZD2=CD2   and  XCYC

(17)

Where:

YC(3)=ZC(3)=0    and     M31=Mα=cosα    −sinα    0sinα       cosα      0

(18)

For D point:

XDYD   and M41 = Mγ= 1         0               00       cosγ      −sinγ

(19)

Final result:

(XO3+XC(3)cosα−XO4)2+(YO3+XC(3)sinα−YO4+ZD(4)sinγ)2+(ZO3−XO4−ZD(4)cosγ)2=CD2

C1D1 is geometrical constraint type SS:

(20)

XC1−XD12+YC1−YD12+ZC1−ZD12=C1D12

Result:

XC 1YC1   and      XD1YD1

(21)

M51=Mα=cosα    −sinα    0sinα       cosα      0    and   M61 = Mγ= 1         0               00       cosγ      −sinγ

Restriction ecuation for C1D1 result:

(XO5−YC1(5)sinα−XO6)2+(YO5+YC1(5)cosα−YO6+ZD1(6)sinγ)2+(ZO5−ZO6−ZD1(6)cosγ)2=C1D12

Kinematic constraints is:			γ = f (t)

(22)

R E F E R E N C E S

[1]	Brine, Gerald, T. Flap System For Short Takeoff and Landing Aircraft , United States Patent US4784355,1988

[2]	Frans Willem de Haan., Cornelis Adrianus.,Aircraft fitted with wing trailing edge flaps actuated by six-bar mechanisms, In: UK Patent Aplication GB 2079688 A, 1982

[3]	Hans Juergen Steinhauer,Walter Martens, Flap Drive SystemFor Airfoils, US4763862, 1988

[4]	Haug, J.E., Computer Aided Kinematics and Dynamics of Mechanical Systems , Allyn and Bacon,U.S.A., 1989

[5] 	Klose, A, J., Airplane Aileron System, In: United States Patent US2369832

[6]	Lawrence,Y,Lam., Michael,Lam.,Aileron For Fixed Wing Aircraft, In: United States Patent US6554229B1,2003

[7]	Lee, Norman. Aircraft Flap System, United States Patent US3785594 , 1972

[8]	Peter K.C. Rudolph , Flap Assembly For Aircraft Wing ,U.S.4381093,1983.

[9]	Shabana, A., Dynamics Of  Multibody Systems, A Wiley Interscience Publication, 1989

[10]	Stephenson, M, F., Wash, Issaquah., Wing Flap Mechanism, In: United States Patent US4605187, 1986

[11]	Visa,I., Alexandru,P., Alexandru,C.,Talaba,D., Proiectarea functionala a mecanismelor ,Metode clasice si moderne, Editura Lux Libris ,Brasov 1998

[12]	Visa,I., Antonya,Cs.,Structural synthesis of planar linkages as multibody systems, Proceedings of the Eighth IFToMM International Symposium on Theory of Machines and Mechanisms Bucharest,Romania Syrom,2001

[13]	Visa,I.,Mechanical Systems Modeling as Multibody Systems in Product Design, Brasov,Romania,PRASIC 2002

[14]	Wang, Timothy., Variable camber leading edge flap, United States Patent US4189120 , 1980

[15]	Westburg,P.W.,Aircraft flap supporting and operating mechanism. United States Patent US3013748

[16]	Deladi , S. , Deladi , L . , Voicu , C. – Indrumar de laborator – Instalatii de comanda si

conducere a aeronavelor Universitatea Transilvania din Brasov 2001

[17]	 Deladi , S. , Deladi , L . , Deliu , Gh. – Mecanica aeronavelor Indrumar de proiect –

Universitatea Transilvania din Brasov 2001