|  | International Workshop Advanced Researches in Computational Mechanics and Virtual Engineering  18 – 20 October 2006, Brasov, Romania |
| --- | --- |

ASPECTS OF THE NON-HOLONOMIC SYSTEMS’ DYNAMICS

Nicolae Pandrea, Nicolae-Doru Stănescu

University of Piteşti, Piteşti, ROMANIA, e-mail: nicolae_pandrea37@yahoo.com

University of Piteşti, Piteşti, ROMANIA, e-mail: s_doru@yahoo.com

Abstract: This paper presents a few aspects of the systems’ motion with non-holonomic constraints. We treat the cases of one or two non-holonomic constraints for a material point and, also, the cases of one or more non-holonomic constraints for the rigid body. In the paper are also presented applications to the concerned subjects.

Keywords: non-holonomic constraints, dynamics, systems, stability

1. INTRODUCTION

Further on, we shall discuss some aspects of the mechanical systems’ motion subjected to non-holonomic constraints.

2. MATERIAL POINT’S DYNAMICS SUBJECTED TO NON-HOLONOMIC CONSTRAINTS

2.1. General aspects

An example of non-holonomic constraint for the material point is the case in figure 1 where the point M describes the plane following curve of the point M1, which has determined motion x1=x1t; y1=y1t.

Figure 1: The material point with a non-holonomic constraint.

If we denote by vthe speed of the point M, one obtains the relations

| xx1−x=yy1−y=vΔ, | (1) |
| --- | --- |

where

| Δ=x1−x2+y1−y2 | (2) |
| --- | --- |

and from here results the non-holonomic constraint given by the relation

| xy1−y−yx1−x=0. | (3) |
| --- | --- |

In the case of the motion of the material point M1 on a curve in the three dimensional space, from the relations as (1), one obtains two non-holonomic constraints defined by the relations

| xy1−y−yx1−x=0; xz1−z−zx1−x=0. | (4) |
| --- | --- |

Generally, the non-holonomic constraints of the material point will be considered in the form

| Ax+By+Cz+D=0, | (5) |
| --- | --- |

where A, B, C, D are functions of x, y, z, t.

2.2. The motion of the material point subjected to one non-holonomic constraint

A. General case

If the material point of mass m is acted by the force

| F=Fxi+Fyj+Fzk | (6) |
| --- | --- |

and it is subjected to the non-holonomic constraint

| Ax+By+Cz+D=0, | (7) |
| --- | --- |

where i, y=x2, k are the versors of the co-ordinates axes, and Fx, Fy, Fz, A, B, C, D are functions of x, X4=ψ, z, t, then the differential equations of the motion are

| dX1dt=X4; my=Fy+λB; mz=Fz+λC, | (8) |
| --- | --- |

λ being a parameter that must be determined.

Deriving the relation (7) with respect to the time and using the notations:

| J=∂A∂x∂A∂y∂A∂z∂B∂x∂B∂y∂B∂z∂C∂x∂C∂y∂C∂z; τ=∂D∂x+∂A∂t∂D∂y+∂B∂t∂D∂z+∂C∂t, | (9) |
| --- | --- |
| Ψ=−xyzJxyzT−τxyzT−∂D∂t, | (10) |

ones obtain the expression

| Ax+By+Cz=Ψ | (11) |
| --- | --- |

where from with the aid of the expressions (8), we deduce the function

| λ=mψ−AFx+BFy+CFzA2+B2+C2. | (12) |
| --- | --- |

To utilize the fourth order Runge-Kutta method to integrate the moving equations, one uses the notations:

| x=x1; y=x2; z=x3, x=x4, y=x5; z=x6 | (13) |
| --- | --- |

and e or she obtains the first order system

| dx1dt=x4; dx2dt=x5; dx3dt=x6; dx4dt=1mFx+λA; dx5dt=1mFy+λB; dx6dt=1mFz+λA. | (14) |
| --- | --- |

In a numerical application, if we consider m=1 kg, Fx=Fy=Fz=0 and the non-holonomic constraint

| x+y+z−3x=0, | (15) |
| --- | --- |

we obtain in order

| J=0; τ=3x00; X6=0; λ=x | (16) |
| --- | --- |

and system (8) becomes

| x=x; y=x; z=z. | (17) |
| --- | --- |

Considering the initial conditions which satisfy the relation (15)

| t=0→x=y=z=0;x=1;y=−1;z=0, | (18) |
| --- | --- |

one obtains the solution

| x=et−1; y=et−2t−1; z=et−t−1. | (19) |
| --- | --- |

B. The theorem of the kinetic energy and mechanical work

Multiplying the relations (8) by x, y, and z, respectively, summing and keeping into account the expression of the elementary mechanical work,

| dL=Fxdx+Fydy+Fzdz, | (20) |
| --- | --- |

one deduces the expression

| mxx+yy+zz=dLdt+λAx+By+Cz | (21) |
| --- | --- |

which can be also written as

| dEc=dL−λDdt, | (22) |
| --- | --- |

where Ec is the kinetic energy,

| Ec=m2x2+y2+z2. | (23) |
| --- | --- |

Expression (22) defines the theorem of the kinetic energy and mechanical work for the case of one non-holonomic constraint. In the case when D=0 and the force F is conservative,

| F=−grad V=−∇V, | (24) |
| --- | --- |

where V represents the potential, one deduces the expression of the mechanical energy conservation

| Ec+V=ct. | (25) |
| --- | --- |

2.3. The motion of the material point subjected to two non-holonomic constraints

If the non-holonomic constraints are

| Aix+Biy+Ciz+Di=0; i=1,2, | (26) |
| --- | --- |

then ones obtain the differential equations system

| YC; my=Fy+λ1B1+λ2B2; mz=Fz+λ1C1+λ2C2. | (27) |
| --- | --- |

Utilizing now the notations:

| Ji=∂Ai∂x∂Ai∂y∂Ai∂z∂Bi∂x∂Bi∂y∂Bi∂z∂Ci∂x∂Ci∂y∂Ci∂z; Gi=∂Di∂x+∂Ai∂t∂Di∂y+∂Bi∂t∂Di∂z+∂Ci∂t, | (28) |
| --- | --- |
| ψi=−xyzJixyzT−GixyzT−∂Di∂t, | (29) |

by derivation with respect to the time, one deduces the expressions:

| Aix+Biy+Ciz=ψi; i=1, 2 | (30) |
| --- | --- |

and hence, from (27), (30), with the aid of the relations:

| Δi=Ai2+Bi2+Ci2; Δ12=A1A2+B1B2+C1C2; Fi=AiFx+BiFy+CiFz, | (31) |
| --- | --- |

we deduce the expressions:

| λ1=Δ2mψ1−F1−Δ12mψ2−F2Δ1Δ2−Δ122; λ2=−−Δ12mψ1−F1+Δ1mψ2−F2Δ1Δ2−Δ122. | (32) |
| --- | --- |

To numerical integration of the differential equations (26) with the parameters λ1, λ2 given by relations (32), we make the notations (13) and results the first order system

| dx1dt=x4; dx2dt=x5; dx3dt=x6; dx4dt=1mFx+λ1A1+λ2A2; dx5dt=1mFy+λ1B1+λ2B2; dx6dt=1mFz+λ1C1+λ2C2. | (33) |
| --- | --- |

Analogously, for the theorem of the kinetic energy and mechanical work, one obtains the expression

| dEc=dL−λ1D1dt−λ2D2dt. | (34) |
| --- | --- |

In a numerical application, if we consider the non-holonomic constraints

| x+y+z−1,5x+y=0; x−y−z+y=0, | (35) |
| --- | --- |

we obtain in order the expressions:

| J1=J2=0; G1=−32110; G2=−110; ψ1=32x+y; ψ2=x−y; Δ1=3; Δ2=2; λ1=x+y2; λ2=x−y2 | (36) |
| --- | --- |

and results, in the absence of the force F, for mass m=1kg, the differential equations:

| x=x; y=y; z=x+y2 | (37) |
| --- | --- |

which, with the initial conditions that satisfy the relations (35):

| t=0→x=y=z=0;x=1;y=1;z=−2, | (38) |
| --- | --- |

have the solution

| x=et−1; y=et−1; z=et−3t−1. | (39) |
| --- | --- |

2.4. Determining the following curve

Let us consider the point M of mass m, acted by the force F (fig. 2), which moves such that its speed v passes through the point M1 with determined motion x1=x1t; y1=y1t; z1=z1t. The equations of motion for the point M in the case of the non-holonomic constraints (4) are:

| mx=Fx+λ1y1−y+λ2z1−z; my=Fy−λ1x1−x; mz=Fz−λ2x1−x. | (40) |
| --- | --- |

Figure 2: The following curve.

It also follows

| ψ1=x1y−y1x; ψ2=x1z−z1x; F1=Fxy1−y−Fyx1−x; F2=Fxy1−y−Fyx1−x; Δ1=y1−y2+x1−x2; Δ12=y1−yz1−z | (41) |
| --- | --- |

and we obtain the system (33), where λ1, v are given by the relations (32).

If the force F is conservative, thenF=gradU and from the relation (34) one deduces the speed v as a function of the speed v0 and the initial position x0,y0,z0, specifically

| v=v02+2mUx,y,z−Ux0,y0,z0. | (42) |
| --- | --- |

From the equalities

| xx1−x=yy1−y=zz1−z=vΔ, | (43) |
| --- | --- |

where

| Δ=x1−x2+y1−y2+z1−z2, | (44) |
| --- | --- |

we obtain the differential equations

| x=x1−xvΔ; y=y1−yvΔ; z=z1−zvΔ, | (45) |
| --- | --- |

which can be numerical integrated with the initial conditions:

| t=0; x=x0; y=y0; z=z0; v=v0; x=x10−x0v0Δ0; y=y10−y0v0Δ0; z=z10−z0v0Δ0; Δ0=x10−x02+y10−y02+z10−z02. | (46) |
| --- | --- |

In a numerical application, we consider

| x1=1; y1=4t; z1=3t; U=2gz | (47) |
| --- | --- |

and results the system (45) where

| v0=4; v=16+2gz;Δ=1−x2+4t−z2+3t−y2. | (48) |
| --- | --- |

The numerical integration leads to the set of values from the table 1.

Table 1: The results of the numerical integration.

| t | x | y | z | x | y | z | v |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 16 | 0 | 0 | 4.000 |
| 0,05 | 1,486 | 0,047 | 0,036 | 19,551 | 1,186 | 0,890 | 4.086 |
| 0,1 | 2,042 | 0,1017 | 0,076 | 7,350 | 1,120 | 0,840 | 4.183 |
| 0,15 | 2,339 | 0,162 | 0,121 | 4,880 | 1,287 | 0,966 | 4.287 |
| 0,2 | 2,550 | 0,230 | 0,173 | 3,697 | 1,453 | 1,090 | 4.403 |
| 0,25 | 2,716 | 0,307 | 0,230 | 2,980 | 1,609 | 1,207 | 4.529 |
| 0,3 | 2,852 | 0,391 | 0,293 | 2,490 | 1,755 | 1,316 | 4.664 |
| 0,35 | 2,967 | 0,482 | 0,362 | 2,131 | 1,894 | 1,421 | 4.805 |
| 0,4 | 3,067 | 0,580 | 0,435 | 1,855 | 2,027 | 1,520 | 4.953 |
| 0,45 | 3,154 | 0,685 | 0,514 | 1,634 | 2,153 | 1,615 | 5.106 |
| 0,5 | 3,231 | 0,795 | 0,597 | 1,453 | 2,276 | 1,707 | 5.263 |
| 0,55 | 3,300 | 0,912 | 0,684 | 1,302 | 2,393 | 1,795 | 5.423 |
| 0,6 | 3,361 | 1,035 | 0,776 | 1,173 | 2,507 | 1,880 | 5.587 |
| 0,65 | 3,417 | 1,163 | 0,872 | 1,061 | 2,616 | 1,962 | 5.754 |
| 0,7 | 3,468 | 1,296 | 0,972 | 0,964 | 2,722 | 2,042 | 5.922 |
| 0,75 | 3,514 | 1,435 | 1,076 | 0,878 | 2,824 | 2,118 | 6.091 |
| 0,8 | 3,556 | 1,578 | 1,184 | 0,801 | 2,922 | 2,192 | 6.262 |
| 0,85 | 3,594 | 1,727 | 1,295 | 0,732 | 3,017 | 2,263 | 6.435 |
| 0,9 | 3,629 | 1,880 | 1,410 | 0,670 | 3,108 | 2,331 | 6.607 |
| 0,95 | 3,661 | 2,038 | 1,528 | 0,614 | 3,195 | 2,396 | 6.780 |
| 1 | 3,690 | 2,199 | 1,650 | 0,564 | 3,278 | 2,459 | 6.954 |

The graphic representations are given in the figures 3, 4 and 5.

From the figures mentioned above and from the table 1 we easily deduce the conclusions: the values of the co-ordinates x, y and z are increasing and tend to the values of x1, y1 and z1; excepting x (because x1 is limited) which tends to zero, the rest of the speeds are increasing, as the module of the speed v, too.

Figure 3: Variations x=xt, y=yt and z=zt for 0≤t≤1s.

Figure 4: Variations x=xt, y=yt and z=zt for 0≤t≤1s.

Figure 5: Variation v=vt for 0≤t≤1s.

3. DYNAMICS OF THE RIGID BODY WITH NON-HOLONOMIC CONSTRAINTS

3.1. The case of one non-holonomic constraint

Let us consider the rigid in motion relative to the reference fix frame OXYZ and let X, Y, Z be the co-ordinates of the mass center C, Cx, Cy, Cz the central principal inertial axes, ψ, θ, ϕ the Eulerian angles, and ωx, ωy, ωz the projections onto the axes Cx, Cy, Cz of the angular speed ω.

The non-holonomic constraint of the rigid is defined by the relation

| AX+BY+CZ+Dψ+Eθ+Fϕ+G=0, | (49) |
| --- | --- |

where A, B, C, D, E, F, G are functions of X, Y, Z, ψ, θ, ϕ, t.

The kinetic energy of the rigid body writes in the form

| Ec=12mX2+Y2+Z2+12Jxωx2+Jyωy2+Jzωz2 | (50) |
| --- | --- |

where m is the mass, Jx, Jy, Jz are the principal central inertial moments, and the angular speeds are given by the relations

| ωx=ψsinθsinϕ+θcosϕ; ωy=ψsinθcosϕ−θsinϕ; ωz=ψcosθ+ϕ. | (51) |
| --- | --- |

If the rigid is acted at C by the force FFx,Fy,Fz and by the moment MMx,My,Mz, then the Lagrange’s equations write in the form

| mX=FX+λA; mY=FY+λB; mZ=FZ+λC, | (52) |
| --- | --- |
| ddt∂Ec∂ψ−∂Ec∂ψ=Mψ+λD; ddt∂Ec∂θ−∂Ec∂θ=Mθ+λE; ddt∂Ec∂ϕ−∂Ec∂ϕ=Mθ+λF, | (53) |

where Mψ, Mθ, Mϕ are the projections of the moment M respectively onto the axis OZ, nodes’ axis and axisCz. It is easy to prove the existence of the relations:

| Mx=Mψsinθsinϕ+Mθcosϕ; My=Mψsinθcosϕ−Mθsinϕ; Mz=Mψcosθ+Mϕ | (54) |
| --- | --- |

and if we use the notations:

| D=Dsinθsinϕ+Ecosϕ; E=Dsinθcosϕ−Esinϕ; F=Dcosθ+F | (55) |
| --- | --- |

we obtain from (52) and from Euler’s conditions, the equations of motion

| mX=FX+λA; mY=FY+λB; mZ=FZ+λC; Jxωx−Jy−Jzωyωz=Mx+λD; Jyωy−Jz−Jxωzωx=My+λE; Jzωz−Jx−Jyωxωy=Mz+λF. | (56) |
| --- | --- |

From the cinematic relations (51) one deduces the expressions:

| ψ=ωxsinϕ+ωycosϕsinθ; θ=ωxcosϕ−ωysinϕ; ϕ=ωz−cosθωxsinϕ+ωysinϕsinθ | (57) |
| --- | --- |

and the equations of the non-holonomic constraint, by using the relations

| D=Dsinϕ+Ecosϕsinθ−Fsinϕcosθsinθ; E=Dcosϕ−Esinϕsinθ−Fcosθcosϕsinθ; F=F | (58) |
| --- | --- |

becomes

| AX+BY+CZ+Dωx+Eωy+Fωz+G=0. | (59) |
| --- | --- |

Deriving this relation and using the relation

| H=−AX+BY+CZ+Dωx+Eωy+Fωz+G, | (60) |
| --- | --- |

one obtains

| AX+BY+CZ+Dωx+Eωy+Fωz=H | (61) |
| --- | --- |

and keeping into account the equations (56), one deuces the expression

| λ=H−AFx+BFy+CFzm−DMx+Jy−JzωyωzJx−EMy+Jz−JxωzωxJyA2+B2+C2m+DDJx+EEJy+FFJz−FMz+Jx−JyωxωyJzA2+B2+C2m+DDJx+EEJy+FFJz. | (62) |
| --- | --- |

To numerical integration, we pass to a first order system using the notations:

| X=X1; Y=X2; Z=X3; ψ=X4; θ=X5; ϕ=X6; X=X7; Y=X8; Z=X9; ωx=X10; ωy=X11; ω=X12 | (63) |
| --- | --- |

and we obtain the system

| dX1dt=X7; dX2dt=X8; dX3dt=X9; dX4dt=X10sinX6+X11cosX6sinX5; dX5dt=X10cosX6−X11sinX6; dX6dt=X12−cosX5sinX5X10sinX6+X11cosX6; dX7dt=Fx+λAm; dX8dt=Fy+λBm; dX9dt=Fz+λCm; dX10dt=Mx+λD+Jy−JzX11X12Jx; dX11dt=My+λE+Jz−JxX12X10Jy; dX12dt=Mz+λF+Jx−JyX10X11Jz, | (64) |
| --- | --- |

where λ calculates with relation (62), and the initial conditions are:

| t=0; X1=X0; X2=Y0; X3=Z0; X4=ψ0; X5=θ0; X6=ϕ0; X7=VX0; X8=VY0;X9=VZ0; X10=ωx0; X11=ωy0; X12=ωz0. | (65) |
| --- | --- |

Based on the calculus relations established above, we can describe the following algorithm for the values Xi, i=1, 2, …, 12 and the known functions AX1,X2,X3,X4,X5,X6, …, GX1,X2,X3,X4,X5,X6.

One calculates in order:

– the functions A, B, C, D, E, F;

– the functions D, E, F, D, E, F with relations (55), (58);

– the derivatives dXidt, i=1, 2, …, 6 with the relations from the set (64);

– the derivatives A, B, C, D, E, F, G with relations like

| A=i=16∂Ai∂XiXi; | (66) |
| --- | --- |

– the derivatives D, E, F with relations

| D=DsinX6+EcosX6sinX5−FsinX6cosX5+dX5dtF−DcosX5sinX5sin2X5+dX6dtE; E=DsinX6−EsinX5sinX6−FcosX5cosX6+dX5dtF−DcosX5cosX5sin2X5+dX6dtD; F=F; | (67) |
| --- | --- |

– the functions H, λ with the relations (61), (62);

– we integrate the system (64) considering the integration step Δt of hundredth of second.

3.2. The case of more non-holonomic constraints

Considering the case when the rigid has p non-holonomic constraints in the form

| AiX+BiY+CiZ+Diψ+Eiθ+Fiϕ+Gi=0, i=1, 2, …, p, | (68) |
| --- | --- |

using the notations

| Di=Disinθsinϕ+Eicosϕ; Ei=Disinθcosϕ−Eisinϕ; F=Dicosθ+Fi, | (69) |
| --- | --- |

one obtains the matrix equations system

| mX=Fx+i=1pλiAi; mY=My+i=1pλiBi; mZ=Mz+i=1pλiCi; Jxωx=Mx+Jy−Jzωyωz+i=10λiDi; Jyωy=My+Jz−Jxωzωx+i=10λiEi; Jzωz=Mz+Jx−Jyωxωy+i=10λiFi. | (70) |
| --- | --- |

This system of equations, with the matrix notations

| M=m000000m000000m000000Jx000000Jy000000Jz; F=FxFyFzMx+Jy−JzωyωzMy+Jz−JxωzωxMz+Jx−Jyωxωy; q=XYZωxωyωzT; A=A1B1C1D1E1F1A2B2C2D2E2F2………………ApBpCpDpEpFp; G=G1G2…GpT; λ=λ1λ2…λpT, | (71) |
| --- | --- |

transforms in the matrix equation

| Mq=F+ATλ. | (72) |
| --- | --- |

Utilizing the notations like (58)

| Di=Disinϕ+Eicosϕsinθ−Fsinϕcosθsin2θ; Ei=Dicosϕ−Eisinϕsinθ−Fcosθcosϕsin2θ; F=F | (73) |
| --- | --- |

the non-holonomic constraints (68) with the aid of relations (57) and the notation

| A=A1B1C1D1E1F1A2B2C2D2E2F2………………ApBpCpDpEpFp | (74) |
| --- | --- |

write in matrix form

| AXYZωxωyωz+G=0. | (75) |
| --- | --- |

Deriving the relation (75) and using the notation

| H=−G−AXYZωxωyωzT, | (76) |
| --- | --- |

one obtains the expression

| Aq=H | (77) |
| --- | --- |

which, keeping into account the relation (72) becomes

| AM−1F+M−1ATλ=H | (78) |
| --- | --- |

and from here we obtain the matrix λ

| λ=AM−1AT−1H−AM−1F. | (79) |
| --- | --- |

Further on, the solution is obtained from the system (72) with λ given by the relation (79).

3.3. The rolling of a wheel on plane

Let us consider a wheel of radius R on a horizontal plane (fig. 6), the wheel being homogenous and rolling on this plane.

Figure 6: The model of a wheel on a plane.

We select the fix frame OXYZ such that the plane OXY coincides to the given plane and we select the mobile frame Cxyz such that the axes Cx, Cy are in the wheel’s plane, C being its center. We shall also consider the system CX'Y'Z' with the axes parallel to those of the system OXYZ. It follows that the node axis CN is parallel to BM, the intersection axis between the plane OXY and the wheel’s plane. In the figure we presented the Euler’s angles: ψ - the precession angle, ϕ - the proper rotation angle, and θ - the nutation angle.

The condition of the rolling without sliding for the wheel on the plane pretends that the speed for the point M of contact between wheel and plane to be zero,

| vC+ω×CM=0, | (80) |
| --- | --- |

where we have applied the general Euler relation for the points C and M.

Due to the fact that the versor u is orthogonal to BM and reads u=sinψi0−cosψj0, where i0, j0 and dX3dt=X6 are the versors of the fix axes, we deduce

| CΜ=Rsinψcosθi0−cosψcosθj0−sinθk0. | (81) |
| --- | --- |

On the other hand, the components ωX, ωY, ωZ of the vector angular speed on the fix frame’s axis are given by

| ωx=ϕsinθsinψ+θcosψ; ωy=−ϕsinθcosψ+θsinψ; ωz=ϕcosθ+ψ. | (82) |
| --- | --- |

Denoting by XC, YC, ZC the co-ordinates of the point C relative to the fix reference system, expression (80) leads us to

| Xc+Rψcosψcosθ−Rθsinψsinθ+Rϕcosψ=0; YC+Rψsinψcosθ+Rθcosψsinθ+Rϕsinψ=0; ZC−Rθcosθ=0. | (83) |
| --- | --- |

The last relation (83) defines a holonomic constraint because it can be integrated leading to

| ZC−Rsinθ=0. | (84) |
| --- | --- |

Therefore, the system has two non-holonomic constraints (the first and the second relation (83)) and a holonomic constraint in differential form (the third relation (83)) or integral form (expression (84)).

Let us consider that the wheel is acted only by its own weight mg.

The position of the wheel is defined by the generalized co-ordinates XC, YC, ψ, θ, ϕ.

The kinetic energy reads

| Ec=12mXC2+YC2+ZC2+12Jxωx2+Jyωy2+Jzωz2, | (85) |
| --- | --- |

the vector ω expressed in the system Cxyz being given by

| ωx=ψsinθsinϕ+θcosϕ; ωy=ψsinθcosϕ−θcosϕ; ωz=ψcosθ+ϕ. | (86) |
| --- | --- |

In addition, Jx=mR24; Jy=mR24; Jz=mR24 and we denote J=Jx=Jy=Jz2.

Results

| Ec=12mXC2+YC2+ZC2+12Jψ21+cos2θ+θ2+2ϕ2+4ψϕcosθ. | (87) |
| --- | --- |

The potential energy writes

| V=mgRsinθ. | (88) |
| --- | --- |

The second order Lagrange’s equations, for the case of non-holonomic constraints:

| k=1haikqk+bi=0; i=1,p, | (89) |
| --- | --- |

with aik, bi, functions of q1, …, qn and t, and conservative forces, are in the forma

| ddt∂Ec∂qk−∂Ec∂qk+∂V∂qk=i=1pλiaik; k=1,h, | (90) |
| --- | --- |

where λi are Lagrange’s multipliers. Ones obtain the expressions

| mXC=λ1; mYC=λ2; Jddtψ1+cos2θ+2ϕcosθ=Rcosθλ1cosψ+λ2sinψ; ddtmR2θcos2θ+Jθ+mR2θ2cosθsinθ+Jψψcosθ+2ϕsinθ=−mgRcosθ+R−λ1sinψ+λ2cosψsinθ ; 2Jddtϕ+ψcosθ=Rλ1cosψ+λ2sinψ. | (91) |
| --- | --- |

Replacing the last relation (91) in the third expression (91), we get

| ψsinθ−2ϕθ=0. | (92) |
| --- | --- |

Deriving the first two relations (83) with respect to time and replacing the results in the last, respectively the second relation (91), one obtains the other two Lagrange’s equations:

| θ=−ψ2sinθcosθ−2J+mR2J+mR2sinθψϕ−mgRJ+mR2cosθ, | (93) |
| --- | --- |
| ϕ=−2ϕθctgθ+2ψθJ+mR22J+mR2sinθ. | (94) |

Denoting now X1=ψ, X2=θ, X3=ϕ, X4=ψ, X5=θ, X6=ϕ, from Lagrange’s equations (92), (93) and (94) ones obtain the six non-linear first order differential equations system

| dX1dt=X4; dX2dt=X5; dX3dt=X6; dX4dt=2X5X6sinX2; dX5dt=−X42sinX2cosX2−2J+mRh2J+mR2X4X6sinX2−mgRJ+mR2cosX2; dX6dt=−2X5X6cosX2sinX2+2J+mR22J+mR2X4X5sinX2. | (95) |
| --- | --- |

The equilibrium positions are determined equating to 0 the right-hand side terms of the relations (95). Results X4=0, X5=0, cosX2=0 and we get the equilibrium positions

| X1=μ1; X2=π2+kπ; X3=μ2; X4=0; X5=0; X6=0, | (96) |
| --- | --- |

where μ1, μ2 are arbitrary real values, and k∈Z.

Let us now denote by fi, i=1,6 the functions in the right-hand sides of the expressions (95) and by jkl=∂fk∂Xl, k=1,6, l=1,6.

The characteristic equation detJ−λI=0, where J is the Jacobi matrix, J=jklk=1,6l=1,6, and I is the unity six order matrix 6, keeping into account that jkl=0 for every k=1,6, l=1,6 excepting j41=1, j25=1,j36=1, j52=mgRJ+mR2, reads

| −λ001000−λ001000−λ001000−λ000j5200−λ000000−λ=0, | (97) |
| --- | --- |

where from λ6=0. The characteristic equation (97) has an unique root λ=0 with the multiplicity order n=6. For the linear system the equilibrium is unstable, the same thing being true for original non-linear system.

In fact, it can be observed that the theorems of the central manifold can not be applied (there not exists the matrix B which has eigenvalues with negative real parts), and a Liapunov function is hard to be found.

The sentence that the equilibrium is unstable is an immediate corollary of theorem Lejeune-Dirichlet. In fact, we shall prove that the equilibrium position given by the expressions (96), in which due to the symmetry of the wheel we limit ourselves to k=0, the potential energy hasn’t a local minimum. Hence, in accord to the relation (88), we have

| V¦θ=π2=mgR; ∂V∂θ¦θ=π2=0; ∂2V∂θ2¦θ=π2=−mgR | (98) |
| --- | --- |

and therefore V has a local maximum at the equilibrium position.

Considering a deviation εit, i=1,6 relative to the solution Xit of the system (95), one obtains the system in variations

| dε1dt=ε4; dε2dt=ε5; dε3dt=ε6; dε4dt=2X5+ε5X6+ε6sinX2+ε2−2X5X6sinX2; dε5dt=−X4+ε42sinX2+ε2cosX2+ε2−−2J+mR2J+mR2X4+ε4X6+ε6sinX2+ε2−−mgRJ+mR2cosX2+ε2+X42sinX2cosX2+2J+mR2J+mR2X4X6sinX2+mgRJ+mR2cosX2 ;; dε6dt=−2X5+ε5X6+ε6ctgX2+ε2++2J+mR22J+mR2X4+ε4X5+ε5sinX2+ε2+2X5X6ctgX2−2J+mR22J+mR2X4X5sinX2. | (99) |
| --- | --- |

Let us consider the motion defined by

| X1=0; X2=π2; X3=0; X4=ω4; X5=0; X6=0, | (100) |
| --- | --- |

i.e. an uniform rotation around a vertical axis that passes through the contact point between the wheel and plane.

System (99) becomes now

| dε1dt=ε4; dε2dt=ε5; dε3dt=ε6; dε4dt=2ε5ε6cosε2; dε5dt=−ω4+ε42cosε2sinε2−2J+mR2J+mR2ω4+ε4ε6cosε2+mgRJ+mR2sinε2; dε6dt=2ε5ε6tgε2+2J+mR22J+mR2ω4+ε4ε5cosε2. | (101) |
| --- | --- |

Making now the evident approximations

| cosε1≈1; sinεi≈εi; εiεj≈0; εi2≈0; i,j=1,6, | (102) |
| --- | --- |

system (101) becomes

| dε1dt=ε4; dε2dt=ε5; dε3dt=ε6; dε4dt=0; dε5dt=ω42ε2−2J+mR2J+mR2ω4ε6+mgRJ+mR2ε2; dε6dt=2J+mR22J+mR2ω4ε5. | (103) |
| --- | --- |

Deriving the relation before the last relation (103) with respect to the time an keeping into account the last relation (103), one finds the expression

| d2ε5dt2=ω42ε5−2ω42ε5+mgRJ+mR2ε5 | (104) |
| --- | --- |

or

| d2ε5dt2+ω42−mgRJ+mR2ε5=0. | (105) |
| --- | --- |

The motion is stable if

| ω42>mgRJ+mR2=ω4critic2. | (106) |
| --- | --- |

REFERENCES

[1] Arrowsmith, D., K., Place, C., M., An Introduction to Dynamical Systems, Cambridge University Press, 1994.

[2] Broer, H., W., Krauskopf, B., Vegter, G., (eds.) Global Analysis of Dynamical Systems, Institute of Physics Publishing, Bristol and Philadelphia, 2001.

[3] Carleson, L., Gamelin, T., W., Complex Dynamics, Springer–Verlag, New York, Heidelberg, London, Paris, 1993.

[4] Carr, J., Applications of Centre Manifold Theory, Springer–Verlag, New York, Heidelberg, Berlin, 1981.

[5] Pandrea, N., Chiroiu, V., Stănescu, N.–D., Munteanu, L., Dynamical systems. Theory an Applications, Romanian Academy Publishing House, Bucharest, 2006 (in press).

[6] Curtu, R., Introduction to the theory of dynamical systems, Infomarket Publishing House, Braşov, 2000.

[7] Dumortier, F., Roussarie, R., Sotomayor, J., Żoladek, H., Bifurcations of Planar Vector Fields. Nilpotent Singularities and Abelian Integrals, Springer–Verlag, Berlin, Heidelberg, New York, 1991.

[8] Georgescu, A., Moroianu, M., Oprea, I., Theory of bifurcation. Principles and applications, Publishing House of the University of Piteşti, 1999.

[9] Guckenheimer, J., Holmes, P., Nonlinear Oscillations, Dynamical Systems, and Bifurcation of Vector Fields, Springer–Verlag, New York, Berlin, Heidelberg, London, Paris, Tokyo, Hong Kong, 1983.

[10] Hale, J., K., Koçak, H., Dynamics and Bifurcations, Springer–Verlag, New York, Berlin, Heidelberg, London, Paris, Tokyo, Hong Kong, Barcelona, Budapest, 1991.

[11] Kuznetsov, Y., A., Elements of Applied Bifurcation Theory, Springer–Verlag, New York, Berlin, Heidelberg, London Paris, Tokyo, Hong Kong, Barcelona, Budapest, 1995.

[12] Lurie, A., I., Analytical Mechanics, Springer–Verlag, Berlin, Heidelberg, New York, Barcelona, Hong Kong, London, Milan, Paris, Tokyo, 2002.

[13] Neĩmark, Ju., I., Fufaev, N., A., Dynamics of Nonholonomic Systems, Translations of Mathematical Monographs, vol. 33, American Mathematical Society, Providence, Rhode Island, 1972.

[14] Pandrea, N., Stănescu, N.–D., Mechanics, Didactical and Pedagogical Publishing House, Bucharest, 2002.

[15] Perko, L., Differential Equations and Dynamical Systems, Springer–Verlag, New York, Berlin, Heidelberg, 1996.

[16] Sibirsky, K., S., Introduction to topological dynamics, Noordhoff International Publishing, Leyden, The Netherlands, 1975

[17] Teodorescu, P., P., Mechanical systems. Classical models, vol. IV, Technical Publishing House, Bucharest, 2002.

[18] Voinea, R., Stroe, I., V., Introduction to the theory of dynamical systems, Romanian Academy Publishing House, Bucharest, 2000.

[19] Pandrea, N., On the motion of the material point with non-holonomic constraints, Scientifical Bulletin of the University of Piteşti, series Applied Mechanics, vol. 1, 1995.