POWER TRANSMISSION ON ELECTRIC CARS   PART 1

Marian VASII 1 ,  Maria Luminita  SCUTARU2

1„Transilvania” University of Brasov, Romania  vasiimarian@yahoo.com

2„Transilvania” University of Brasov, Romania        lscutaru@unitbv.ro

Abstract  The electric gear-boxes  have  a continue variation  of conduction ratio  and are composed from  electric generator  and one or more  electric engines. The mechanical power produced by  the engine with internal burning is transform in electric energy by the  electric generator  and this will be transform again   in mechanical energy  by  the one or more electric engines. When the electric motor are  in the wheels  of the  car  is not required the transmission of cars  namely  main transmission  and differential because in this case is an electric transmission. If the motion from the electric motor is transmited  to wheels by mechanical transmission will be  speak about the  electromechanical transmission.

Keywords : electric cars ;  transmission ; electric generator

1. INTRODUCTION

The electric transmission  present  some advantage :

- assure it a variation continuously of transmission ratio

- between certain limit assures the automatic accommodation of the car to conditions of road

- is the most quick and precise way of control

- assure the engine and transmission   durability  because of the smooth variations regims of upload and the absence of rigid connection among motor and transmission

- the electric motor in wheels permits the placement of the engine with internal burning in any place inside the  car

- feature of traction for electric motors  of continuous current is very favorably for cars

Among the disadvantage is remembered:

- assure a low efficiency than the mechanical transmissions

- weight and big gauge; the specifies weight of all  the elements in a electromechanical transmission represent 68….136 N/Kw,  against the specifies weight  of the mechanical elements replaced which is about 7…40 N/Kw

- are much expensive

Fig  1  Features of couple limits for different  electric motors

a-continuous current with series excitation,  b-continuous current with paralel excitation,  c- asynchronous, d-synchronous with permanent excitation,    Ibmax, Ismax – maximum current for battery respectively control, Pb limit- maximum batteries, Ps – the power of the  control uni

Fig 2  Curve of izorandament for electric motor of direct current with parallel excitation

Characteristic of traction of direct current motor with parallel excitation is very  favorable and this feature is  very  much spred among the cars  which utilize the electric motor for traction . For illustrate the way wich  efficiensy depends on  revolution and the torque value in figure 2 is prezented the curve of izorandament  of electric motor of direct current with  parallel excitation.

Fig 3  Transmission  scheme of  a car with a single traction engine

Electric scheme of a electromechanical transmission with a single electrical engine is show in the figure 3.

The engine with internal burning M is powering the generator 7, having the winding-up of excitation 10  connected in parallel  .A loop  of excitation  winding-up is connected in series. A part of excitation winding-up  cant be shortcircuit by relays 14,15 and 16. With the help of the switch 12  commanded by  acceleration pedal of the engine with intenal burning in the circuit of excitationis is put the storage batteries 13. The electric motor of traction 6 has excitation winding-up 5 connected in series .The electromagnetic fluxes sense  from excitation  winding-up 5 and therefore the sense  of rotation of  indus   can be modify with the help of inversor 4, manually commanded.

Variation in large limit of funtion regims of the electric motor  causes an unimportant modification of generator revolution and  implicit of motor revolution with internal burning . To usual scheme the revolution modification and torque  with  500...600%  produces a varition a generator revolution  of only  5... 10%.   This absence of  adaptability of the power of road condition causes the increase of fuel . For to remove this trouble , in electric scheme they entered the relays 14,15 and 16 commanded by coil 1,2 and 3 connected in series  with excitation coil of the electric motor  from that is establish a dependency between generator excitation and curent  amperage   from  the winding-up of the electric motor excitation.

Fig. 4 Characteristic of traction  of a car with electromechanical transmission in two stages: Fnz – total resistant to go forward; Ibmax ,Ismax. – the maximum currents for battery respectively controller,; PBlimit  the maximum power of battery ; Ps ~the pqwer of the controller

In the figure 4 is presented characteristic of traction of one vehicule   with electromechanical transmission  in two stage .The gear-boxes in two stage  allows a better adaptation of  car transmission on road condition  . In first gear  this   ensurea   the maximum value of tractiune force, and in second gear  the maximum of speed.

Fig. 5 Constructiv solution of electromechanical with one electrical  enginer (Toyota E-com)

More electrical engine

On electrical cars the electrical power  is maid  directly from the  chemichal energy  storegy  in the battery .In caz  of this type  of cars  is not needed to  talk about a gear-boxesbut about a electrical  transmission.

Fig. 6  Possibility of connect electrical engine:   a - series; b -parallel; c - both

In the figure 6 is shown the possibility of connect electrical engine (represented through circle)   to electric circuit of generator G in case of the vehicle with four wlheels drive.

CONCLUSION

The cars with electrical transmission is better from different point of view , from the point of  view of building  is very  easy but is very expensive for now but in the future the price could be smaller from the point of view of  smoke is very  environmentally friendly.

BIBLIOGRAPHY

[1] Ambrosi,G. Orofino,L. Driveline vibration simulation in a four-wheel drive vehicle. In: 925088 XXIV FISITA Congress, part 2, p. 105-115, Londra, 1992.

[2] Baines,N.C. PantingJ.R. Etemad,M.R. Besant C.B. A gas turbine-electric vehicle concept. In: 925080 XXIV FISITA Congress, part 2, p. 27-32, Londra, 1992.

[3] Baker,A. Torotrak Transmission. In: Automotive Engineer Volume 21 Number 3 June/July 1996, p.46, 47.

Baselga,S. Miravete,A. Pellicer.E. Alba,J. Cdlculo de la cadena de transmision y de las prestaciones del automovil. C.P.S. de la Universidad de Zaragoza, 1995.

[4] Brusaglino,G. Vittone.O. Traction system for electric vehicles. In: 925081 XXIV FISITA Congress, part 2, p. 33-46, Londra, 1992.

[5] Burger,A. Gierer,G. Haupt,J. VolkelJ. Das neue Fiinfgang-Automatik-getriebe fur die neue BMW 3er Baureihe. In: ATZ und MTZ, 1998, p. 134-140.

[6] Chi§u,E. Calianu, D. Velicu,D. Moldovean,Gh. Uber die Synthese der stufenlosen Kraftfahrzeugschalt-getriebe. In: CONAT Vol.11, p.323-329, Bra§ov, 1993.

[7] Ciolan,Gh. Preda,I. Salajan,C. Vulpe,V. Studiu privind solicitdrile transmisiei unui autoturism ARO-10 in regim tranzitoriu. In: Buletinul ESFA, vol.Ill, p.85-88, Bucure§ti, 1995.

[8] CiolaiijGL Preda,I. Dogariu,M^ Determindri experimental ale solicitdrilor unor organe ale transmisiei. In: Buletinul CONAT, vol.IV, p.325-330, Brasov, 1996.

[9] Coanda,S. PredaJ. Ciolan,Gh. Multiplexing sistem for cars. In: 6st EAEC (European Automobile Engineers Cooperation) European Congress "Lightweight and Small Cars - The Answer to Future Needs" Proceedings, volume II, p.999-1002, Cernobbio, 1997.