|  | International Workshop Advanced Researches in Computational Mechanics and Virtual Engineering  18 – 20 October 2006, Brasov, Romania |
| --- | --- |

ON THE FOLLOWING CURVE IN SPACE

Nicolae Pandrea, Nicolae-Doru Stănescu

University of Piteşti, Piteşti, ROMANIA, e-mail: nicolae_pandrea37@yahoo.com

University of Piteşti, Piteşti, ROMANIA, e-mail: s_doru@yahoo.com

Abstract: In this paper we consider the general case of a particle with non-holonomic constraints. We obtain the moving equation system and we discuss its solving both for a conservative and non-conservative force. Finally we present a numerical application.

Keywords: non-holonomic constraints, motion

1. INTRODUCTION

Let us consider a particle of mass macted by the force F. The speed of the particle is always directed to a point situated on a curve C2 by know parametric equations X=Xt, Y=Yt, Z=Zt. At the moment t=0, the particle is situated in the point x=x0, y=y0, z=z0.

Our aim is to determine the curve C1 (the curve described by the particle) and the law of its motion.

Figure 1: The model of a particle with non-holonomic constraints.

2. ESTABLISHING THE LAGRANGE’S SECOND ORDER EQUATIONS

Let us consider that between the generalized co-ordinates of the system there exist p non-holonomic constraints. These constraints can be written as:

| k=1haikqk+bi=0; i=1,2,...,p, | (1) |
| --- | --- |

where aik, bi are functions of q1, q2, …, qn and t.

The relations (1) can also be written as:

| k=1haikdqk+bidt=0; i=1,2,...,p. | (2) |
| --- | --- |

Due to the fact that the virtual displacements don’t depends on time, from (2) results

| k=1haikδqk=0. | (3) |
| --- | --- |

On the other hand, basing on the virtual work principle, we have

| j=1nFj−mjajδrj=0 | (4) |
| --- | --- |

and, because

| δrj=k=1h∂rj∂qkδqk, | (5) |
| --- | --- |

from (4), interchanging the summing order, results

| k=1hj=1nFj−mjaj∂rj∂qkδqk=0. | (6) |
| --- | --- |

Denoting

| Qk=j=1nFj∂rj∂qk | (7) |
| --- | --- |

and keeping into account the relations:

| ∂rj∂qk=∂vj∂qk; ddt∂rj∂qk=∂vj∂qk, | (8) |
| --- | --- |

one obtains the identity

| j=1nFj−mjaj∂rj∂qk=Qk−ddt∂Ec∂qk+∂Ec∂qk | (9) |
| --- | --- |

and the expression (6) becomes

| k=1hQk−ddt∂Ec∂qk+∂Ec∂qkδqk=0, | (10) |
| --- | --- |

where the virtual displacements δqk are no longer independents, because they also must fulfill the conditions (2).

Multiplying the relations (3) by parameters λi, respectively, (the Lagrange’s multipliers) and summing these relations, by adding this sum to relation (10), results the expression

| k=1hQk−ddt∂Ec∂qk+∂Ec∂qkδqk+i=1pλik=1haikδqk=0, | (11) |
| --- | --- |

or

| k=1hQk−ddt∂Ec∂qk+∂Ec∂qkδqk+i=1pk=1hλiaikδqk=0 | (12) |
| --- | --- |

and, by interchanging the summing order at the second sum, we obtain

| k=1hQk−ddt∂Ec∂qk+∂Ec∂qk+i=1pλiaikδqk=0. | (13) |
| --- | --- |

According to the theory of the Lagrange’s multipliers, in the system obtained by adding the parameters λi, the displacements δqk become independents and thus by equating to zero the coefficients of these displacements, it follows:

| Qk−ddt∂Ec∂qk+∂Ec∂qk+i=1pλiaik=0, where k=1,2,...,h, | (14) |
| --- | --- |

or

| ddt∂Ec∂qk−∂Ec∂qk=Qk+i=1pλiaik, where k=1,2,...,h, | (15) |
| --- | --- |

obtaining thus the Lagrange’s equations in the case of non-holonomic constraints.

If there exist conservative forces, too, we call the potential V of these forces and the generalized forces Qk corresponding to the non-conservative forces, and the Lagrange’s equations for this case read:

| ddt∂Ec∂qk−∂Ec∂qk+∂V∂qk=Qk+i=1pλiaik, where k=1,2,...,h. | (16) |
| --- | --- |

At these equations we add the non-holonomic constraints conditions (1) and we obtain a system of h+p differential equations from which is possible to determine the unknowns q1, q2, …, qh, λ1, λ2, …, λp.

3. THE EQUATIONS FOR THE PARTICLE

For our problem we obtain the equations

| mx=Fx+λ1Y−y+λ2Z−z; my=Fy−λ1X−x; mz=Fz−λ2X−x. | (17) |
| --- | --- |

Multiplying the first relation (17) by dx, the second relation (17) by dy, and the third relation (17) by dz, and summing the results, one obtains

| dmv22=Fxdx+Fydy+Fzdz. | (18) |
| --- | --- |

If F is non-conservative, we apply the general theory described in the paragraph 2.

If F is conservative, we obtain

| v=2mUx,y,z−Ux0,y0,z0, | (19) |
| --- | --- |

where U is the function of force that generates the force F.

Relations (17) can be written in the equivalent form

| xX−x=yY−y=zZ−z. | (20) |
| --- | --- |

Using the notation

| Δ=X−x2+Y−y2+Z−z2, | (21) |
| --- | --- |

from relation (20), result:

| dxdt=vΔX−x; dydt=vΔY−y; dzdt=vΔZ−z. | (22) |
| --- | --- |

4. APPLICATION

We consider a numerical application for which

| X=1; Y=4t; Z=3t; U=2gz | (47) |
| --- | --- |

and results the system (22) with

| v0=4; v=16+2gz;Δ=1−x2+4t−z2+3t−y2. | (48) |
| --- | --- |

Numerical interation leads us to the following table.

Table 1: Results of the numerical integration.

| t | x | y | z | x | y | z | v |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 16 | 0 | 0 | 4.000 |
| 0,05 | 1,486 | 0,047 | 0,036 | 19,551 | 1,186 | 0,890 | 4.086 |
| 0,1 | 2,042 | 0,1017 | 0,076 | 7,350 | 1,120 | 0,840 | 4.183 |
| 0,15 | 2,339 | 0,162 | 0,121 | 4,880 | 1,287 | 0,966 | 4.287 |
| 0,2 | 2,550 | 0,230 | 0,173 | 3,697 | 1,453 | 1,090 | 4.403 |
| 0,25 | 2,716 | 0,307 | 0,230 | 2,980 | 1,609 | 1,207 | 4.529 |
| 0,3 | 2,852 | 0,391 | 0,293 | 2,490 | 1,755 | 1,316 | 4.664 |
| 0,35 | 2,967 | 0,482 | 0,362 | 2,131 | 1,894 | 1,421 | 4.805 |
| 0,4 | 3,067 | 0,580 | 0,435 | 1,855 | 2,027 | 1,520 | 4.953 |
| 0,45 | 3,154 | 0,685 | 0,514 | 1,634 | 2,153 | 1,615 | 5.106 |
| 0,5 | 3,231 | 0,795 | 0,597 | 1,453 | 2,276 | 1,707 | 5.263 |
| 0,55 | 3,300 | 0,912 | 0,684 | 1,302 | 2,393 | 1,795 | 5.423 |
| 0,6 | 3,361 | 1,035 | 0,776 | 1,173 | 2,507 | 1,880 | 5.587 |
| 0,65 | 3,417 | 1,163 | 0,872 | 1,061 | 2,616 | 1,962 | 5.754 |
| 0,7 | 3,468 | 1,296 | 0,972 | 0,964 | 2,722 | 2,042 | 5.922 |
| 0,75 | 3,514 | 1,435 | 1,076 | 0,878 | 2,824 | 2,118 | 6.091 |
| 0,8 | 3,556 | 1,578 | 1,184 | 0,801 | 2,922 | 2,192 | 6.262 |
| 0,85 | 3,594 | 1,727 | 1,295 | 0,732 | 3,017 | 2,263 | 6.435 |
| 0,9 | 3,629 | 1,880 | 1,410 | 0,670 | 3,108 | 2,331 | 6.607 |
| 0,95 | 3,661 | 2,038 | 1,528 | 0,614 | 3,195 | 2,396 | 6.780 |
| 1 | 3,690 | 2,199 | 1,650 | 0,564 | 3,278 | 2,459 | 6.954 |

The graphics are rawn in figures 2, 3, and 4.

From the mentioned figures and from table 1, one easily deduces the conclusions: the co-ordinates x, y and z have increasing values, which tend to the values of X, Y and Z; excepting x (due to the fact that X is limited) that tends to zero, the other speeds have increasing values as is the modul of the speed v, too.

Figure 2: Variations x=xt, y=yt and z=zt for 0≤t≤1s.

Figure 3: Variations x=xt, y=yt and z=zt for 0≤t≤1s.

Figure 4: Variation v=vt for 0≤t≤1s.

5. CONCLUSIONS

In our paper we present the general theory to obtain the second order Lagrange’s equations for the most general case of the non-holonomic constraints. We deduce the equations of motion for the particular case of a particle. Finally, a numerical application was presented and the characteristic diagrams were captured.

REFERENCES

[1] Arrowsmith, D., K., Place, C., M., An Introduction to Dynamical Systems, Cambridge University Press, 1994.

[2] Broer, H., W., Krauskopf, B., Vegter, G., (eds.) Global Analysis of Dynamical Systems, Institute of Physics Publishing, Bristol and Philadelphia, 2001.

[3] Carleson, L., Gamelin, T., W., Complex Dynamics, Springer–Verlag, New York, Heidelberg, London, Paris, 1993.

[4] Carr, J., Applications of Centre Manifold Theory, Springer–Verlag, New York, Heidelberg, Berlin, 1981.

[5] Pandrea, N., Chiroiu, V., Stănescu, N.–D., Munteanu, L., Dynamical systems. Theory an Applications, Romanian Academy Publishing House, Bucharest, 2006 (in press).

[6] Curtu, R., Introduction to the theory of dynamical systems, Infomarket Publishing House, Braşov, 2000.

[7] Dumortier, F., Roussarie, R., Sotomayor, J., Żoladek, H., Bifurcations of Planar Vector Fields. Nilpotent Singularities and Abelian Integrals, Springer–Verlag, Berlin, Heidelberg, New York, 1991.

[8] Georgescu, A., Moroianu, M., Oprea, I., Theory of bifurcation. Principles and applications, Publishing House of the University of Piteşti, 1999.

[9] Guckenheimer, J., Holmes, P., Nonlinear Oscillations, Dynamical Systems, and Bifurcation of Vector Fields, Springer–Verlag, New York, Berlin, Heidelberg, London, Paris, Tokyo, Hong Kong, 1983.

[10] Hale, J., K., Koçak, H., Dynamics and Bifurcations, Springer–Verlag, New York, Berlin, Heidelberg, London, Paris, Tokyo, Hong Kong, Barcelona, Budapest, 1991.

[11] Kuznetsov, Y., A., Elements of Applied Bifurcation Theory, Springer–Verlag, New York, Berlin, Heidelberg, London Paris, Tokyo, Hong Kong, Barcelona, Budapest, 1995.

[12] Lurie, A., I., Analytical Mechanics, Springer–Verlag, Berlin, Heidelberg, New York, Barcelona, Hong Kong, London, Milan, Paris, Tokyo, 2002.

[13] Neĩmark, Ju., I., Fufaev, N., A., Dynamics of Nonholonomic Systems, Translations of Mathematical Monographs, vol. 33, American Mathematical Society, Providence, Rhode Island, 1972.

[14] Pandrea, N., Stănescu, N.–D., Mechanics, Didactical and Pedagogical Publishing House, Bucharest, 2002.

[15] Perko, L., Differential Equations and Dynamical Systems, Springer–Verlag, New York, Berlin, Heidelberg, 1996.

[16] Sibirsky, K., S., Introduction to topological dynamics, Noordhoff International Publishing, Leyden, The Netherlands, 1975

[17] Teodorescu, P., P., Mechanical systems. Classical models, vol. IV, Technical Publishing House, Bucharest, 2002.

[18] Voinea, R., Stroe, I., V., Introduction to the theory of dynamical systems, Romanian Academy Publishing House, Bucharest, 2000.

[19] Pandrea, N., On the motion of the material point with non-holonomic constraints, Scientifical Bulletin of the University of Piteşti, series Applied Mechanics, vol. 1, 1995.

[20] Pandrea, N., Stănescu, N.-D., Aspects of the non-holonomic systems’ dynamics, International Workshop Advanced Researches in Computational Mechanics and Virtual Engineering , 18 – 20 October 2006, Brasov, Romania