BULLETIN OF THE TRANSILVANIA UNIVERSITY OF BRAŞOV

## OPTIMISATION  OF  THE  AXIAL  PROPULSION  FORCE

M. D. CAZACU*

Abstract: One presents the theoretical principles of an original method for the propeller design, to obtain the maximum axial propulsion force of a ship or aircraft screw. This method of the axial propulsion force maximisation, exerted by an axial turbo-machine rotor, considers the maximisation of the ratio between the axial force and the mechanical power necessary to the rotor driving. The method give the possibility to determine the optimum shape of the blade profile and its optimum-settling angle for different blade radii.

Keywords: propulsion force.

1. The importance of the research concerning the optimisation methods of hydraulic

and aerodynamic turbo-machine design

Taking into consideration the special importance of a good efficiency of the great number of existent hydraulic and aerodynamic machines, both regarding the energy input by the turbines and the economy of consumed energy by the pumps, ventilators, blowers and turbo-compressors, I developed in the last years an original method to maximise the axial turbo-machine performances, by its applying in the following three design fields:

- the maximisation of the mechanical power, extracted by the hydraulic or wind turbine from the kinetic energy of fluid current [1][2],

- the maximisation of the axial force developed by a hydraulic or aerodynamic screw, realised unconditioned [3] or reported to the consumed power, as in the present paper, and in a future work,

- the maximisation of the fluid current velocity produced by an axial turbo-machine rotor of a fan or a pump.

Encouraged by the unexpected good experimental results [1][2], concerning the application of the method of mechanical power maximisation at the hydraulic or wind turbine shaft, extracted from the kinetic energy of the fluid current, I try in the present to apply these results for the propulsion of the small environmental friendly ships, using the solar energy source by means of photo-voltaic panels and which are foreseen to sail in our biosphere reservation Danube Delta and for ecological tourism [4][5], very important not only for the operation radius enlargement of an ship or aircraft, but also for the fossil fuel savings and environmental protection. This is also of a greatest importance for the ecological boats using the solar energy [6].

- The maximisation of the axial force reported to the driving mechanical power

Starting from the classical theory and practice of the airfoil, one presents in the figure

number 1 the lift and drag components (1), exerted on a profiled hydraulic machine blade or on an aircraft wing in the relative flow, defined by the velocity triangle constituted by the fluid absolute velocity C, fluid relative velocity W and the transport velocity U =R.

,   	.	      	        (1)

- Projecting these forces, for example in the case of the blade peripheral profile, both on the axial direction and on the peripheral direction of the profile motion, we can write the expression of the axial force exerted at the turbo-machine shaft

- ,	        (2)

relative angle - β

blade angle  β b = β + i

W = V/sin β – relative fluid velocity

U = R ω = V/tg β

Fy cos β		     Fx sin β

Fx – the profile drag component

the profile lift component - Fy

Fy sin β

i – profile incidence angle

fluid absolute velocity V

Fig. 1. The velocity triangle and  the hydrodynamic resultant components

and also the expression of the shaft driving mechanical power

.	          (3)

To obtain the maximum value of the axial force in the condition of minimum driving power, we shall calculate the dimensionless ratio

(4)

and cancelling its partial differential with respect to the relative angle β, included between 0 ≤ β ≤ π/2

,      (5)

one obtain the maximisation condition, that by introducing the notation x = ctg β, led as to the solving of the algebraic equation of 2nd degree, having the coefficients as function of the profile fineness and its incidence angle i, considered with respect of the relative velocity W

, 			       (6)

having always two real solutions, one positive and other negative

,				   (7)

as one can see from the juxtaposed table nr. 1, for the case of Göttingen 450 profile [1], which are vindicated again as the best performing, and where we put also the value of the ratio (4) for the confirmation of the maximal value of the axial force, obtained at the approximate incidence angle  i ≈ 1o (fig. 2).

Table 1

| i (degrees) | Cy (i) | Cx (i) | f = Cy / Cx | β (degrees) | fa / pm |
| --- | --- | --- | --- | --- | --- |
| -3 | 0,20 | 0,023 | 8,696 | 41,74 | 0,7949 |
| 0 | 0,41 | 0,02 | 20,5 | 43,63 | 0,9071 |
| 3 | 0,63 | 0,032 | 19,688 | 43,57 | 0,9034 |
| 6 | 0,85 | 0,055 | 15,455 | 43,27 | 0,8887 |
| 9 | 1,05 | 0,081 | 12,963 | 42,82 | 0,8572 |
| 12 | 1,15 | 0,112 | 10,268 | 42,24 | 0,8233 |
| 15 | 1,21 | 0,147 | 8,231 | 41,56 | 0,7848 |

Fig. 2. Dependence with the profile incidence of the axial force reported to the consumed driving mechanical power

the peripheral optimum relative angle having the approximate value βp ≈ 43,65o  (fig. 3)

Fig. 3. The value of the peripheral relative angle βp that realise the maximisation of axial force reported to the driving power as function of profile incidence angle i

In the figure number 4 we give the variation of Gö 450 profile fineness with the incidence angle i.

Fig. 4. Göttingen 450 profile fineness as function of the attack angle i

# 3. Determination of the optimum profile setting angle for other blade radii

Because for the other blade radii Rj, the relative angle βj is already determined by the formula

,	(8)

and taking into consideration the relations that give the expressions of lift and drag coefficients in the case of profile Gö 450 [7], in which we considered the variation of profile lift coefficient as function of the attack angle i, named now of incidence

(9)

and the parabolic variation with the incidence angle of the drag coefficient

, 	         (10)

we will maximise the axial force reported to the driving mechanical power (4), given in this case by the expression in which we assumed the blade portions of wing spread to bee bj = R = constant on the radius, as well as the wing depths  lj

,		       (11)

By cancelling its partial differential with respect to the incidence angle i and dividing this expression with its common factor (1 + ctg2 j)  0, we obtain the equation of 2nd degree

,	      (12)

or with (9) and (10)

,            		      (12’)

those two solutions:  i1 = 0.067  0  maximise the ratio between the axial force and the consumed driving power, while  i2 = - 9.6834  diminishes this ratio to the zero value, being the same for any blade profile radius.

In the table number 2 we give the values of profile relative angle  j  b  for any bade radii  rj  1.

Table 2

| rj | 1.0 | 0.9 | 0.8 | 0.7 | 0.6 | 0.5 | 0.4 | 0.3 | 0.2 | 0.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| j(o) | 43.65 | 46.67 | 50.02 | 53.73 | 57.83 | 62.34 | 67.25 | 72.54 | 78.16 | 84.02 |

4. Conclusions

The interesting results obtained by this method of propeller design optimization, applied to the formerly vindicated Göttingen 450 profile [2], give us now the possibility to apply this method even to other profiles, to find maybe a better blade shape and after this to realize a laboratory model of such screw to demonstrate also experimentally the value of this method.

# References

1. Cazacu M.D., Nicolaie S.: Micro-hydro-turbine for run-of-river power station. The 2nd  Conference of Romanian Hydro-energeticians, 24–25 May 2002, University Politehnica, Bucharest,Vol. II, 443-448.

2. Cazacu M.D., Băran Gh., Nicolaie S.: Încercarea în laborator a microturbinei pentru asigurarea autonomiei energetice a balizelor luminoase pe Dunăre. [The laboratory test of the micro hydro turbine for the energetic autonomy assurance of the lighting buoys on Danube]. International Conference on Energy and Environment – CIEM 03, 23-25 Oct.2003, University Politehnica, Bucharest, Section 3 - Hydroenergetics, Session 2 – Hydraulic Machines and Equipment’s. Hydro-generators, 119-124

3. Cazacu M.D.: The maximisation of the propulsion force for an aircraft or ship propeller. The 30th “Caius Iacob” Conference on Fluid Mechanics and its Technical Applications, Bucharest., 25-26 November 2005, CD.

# 4. Cazacu. M.D.: Microagregat hidroelectric pentru asigurarea autonomiei energetice a balizelor luminoase sau a unor bărci fluviale. [Hydroelectric micro unit for the energetic autonomy assurance of the lighting buoys or of any river boats]. Rev. Ştiinţă, Industrie, Tehnologie, Nr.2, Bucharest. 2005, 44 - 45.

5. Tudor G., Mocanu Z.: Solar energy propelled craft intended for the ecological tourism and transport in the “Danube Delta Biosphere” and other protected zones. The 1st International Symposium “Renewable Energies and Sustainable Development”, 23-25 September, 2004, Danube Delta-Tulcea, Romania.

# 6. Aucouturier J.L., Henry H., Stempin E.: Electro solar boats for passengers transport: a reality now. The 2nd International Symposium “Renewable Energies and Sustainable Development”, 22-24 September, 2005, Danube Delta-Tulcea, Romania.

7. Hütte I.: Manualul inginerului [Engineer’s Textbook], Vol. I, Romanian Engineer’s General Association Publishing House, Bucharest, 1947, p.511.

OPTIMIZAREA  FORŢEI  AXIALE  DE  PROPULSIE

Rezumat: Se prezintă principiile teoretice a unei metode originale de proiectare a elicii, pentru a se obţine forţa axială maximă de propulsie pentru o navă sau un avion.  Această metodă de maximizare a forţei axiale de propulsie, exercitată de un rotor axial de turbomaşină, consideră maximizarea raportului dintre forţa axială şi puterea mecanică necesară antrenării rotorului. Metoda dă posibilitatea determinării formei optime a profilului palei şi a unghiului lui de aşezare pentru diferite raze ale palei.

Cuvinte cheie: forţa de propulsie.

* Hydraulics and hydraulic Machines Department,  POLITEHNICA University of Bucharest.