|  | 1st International Conference Computational Mechanics and Virtual Engineering  COMEC 2005 20 – 22 October 2005, Brasov, Romania |
| --- | --- |

MODEL OF A DYNAMICALLY TUNED GYROSCOPE

Diana Cotoros1, Dumitru Nicoara1

1 Transilvania University, Brasov, ROMANIA,  e-mail dcotoros@unitbv.ro

Abstract :  The paper aims at generating a model for a dynamically tuned gyroscope, provided with  two inner rotors, connected at 900. Firstly the model is represented at rest, using mathematical functions within  the  Mathcad programme. It is necessary to create representations for both the rotors and also for  the torsion bars linking the rotors and the rotors to  the gyroscope spinning axis. Then we may introduce a rotation angles matrix and changing the  mathematical  functions accordingly, we get the visualization of the inner rotors displacements and also of the torsion bars. The rotation angles matrix may be changed according to the parameters to be studied and the programme offers immediately the new positions of the inner rotors. Their displacement provides valuable information for the gyroscopice devices, which equip all types of aircrafts or even landvehicles, such as gyrocompasses or directional gyros.

Keywords:  dynamically tuned gyroscope, rotor, torsion bars

1. INTRODUCTION

A dynamically tuned gyroscope consists of a number of rotors connected to a spinning shaft and one to each other. The link between the rotors is represented by a suitable number of torsion bars, which ensure small displacements of the rotors. They allow measuring two perpendicular components of the gyroscope frame angular velocity with respect to the inertial frame. In order to be dynamically tuned, the parameters of the torsion bars and inner rotors and also the magnitude of the rotor angular velocity should be chosen so that the torsion stiffness of the bars compensate the rotor dynamical swinging moment.

According to the device it is used for (gyrocompass, directional gyro, angular velocity gyro, etc.), the gyroscope may consist of two or three rotors, having two or three degrees of freedom. Before assembling the gyroscopic devices on the aircraft, they should be tested, among others, for the stability of their indications. Usually the experimental measurements are expensive and take much time and accuracy. This is why it appears to be necessary to create a virtual model on the computer and visualize the rotors displacements.

2. MATHCAD REPRESENTATIONS

The present paper deals with the two rotors dynamically tuned gyroscope, which looks like the one in Fig.1.

Figure 1: Dynamically tuned gyroscope

z

x

y

1

7

5

6

4

3

2

The device is provided with two transducers 4 and 5, one for angles and the other one for couples measurement. The shaft 3 is set into motion by help of an electric motor 6, while 1 and 2 represent the inner rotors, linked with perpendicular torsion bars.

First we are going to generate, using Mathcad and suitable mathematical functions the model at rest. The matrices corresponding to the rotations around axes Ox, Oy and Oz are the following:

Rx(u)=1000cos(u)−sin(u)0sin(u)cos(u)								(1)

Ry(u)=cos(u)0−sin(u)010sin(u)0cos(u)								(2)

Rz(u)=cos(u)−sin(u)0sin(u)cos(u)0001								(3)

The circles representing the shaft (cross – section) and the two inner rotors are generated by help of the following functions:

pj=Rzj⋅π200									(4)

p1j=3Rzj⋅π200									(5)

p2j=5Rzj⋅π200									(6)

The torsion bars are represented according to the following expressions, allowing the modelling of a perpendicular connection between the rotors:

qj=Rzπ20⋅−1j+AT								(7)

q1j=Rzπ20⋅−1j−AT								(8)

sj=Rz(0)0⋅−1j+BT									(9)

s1j=Rz(0)0⋅−1j−BT								(10)

In fig.2 we obtained the representation of the model at rest.

Figure 2: Model at rest

We create now a matrix containing the angles values corresponding to the rotors rotations due to the deformation of the torsion bars.

For example

φ=1540−40−305030⋅π180									(11)

The rotation is given by

R(j)=Ry(φ2,j)⋅Rx(φ1,j)⋅Rz(φ0,j)								(12)

The new positions of the inner rotors are generated using the following functions:

P1j=R(0)⋅p1j+A1T									(13)

P2j=R(1)⋅p2j+A1T									(14)

The corresponding positions of the torsion bars are simulated by help of the following expressions:

Qj=R(0)⋅qj+A1T									(15)

Q1j=R(0)⋅q1j+A1T									(16)

Sj=R(1)⋅sj+A1T									(17)

S1j=R(1)⋅s1j+A1T									(18)

The matrices A, B and A1, used in equations (7) – (10) and (13) – (18) are expressing the dimensions for the rotors and the torsion bars, adjusted to those encountered in various gyroscopic devices.

The new positions of the inner rotors and of the torsion bars can be visualized in fig.3.

Figure 3: First visualization of the rotors and torsion bars

In order to check again the model we will try some new positions given by the following angles:

φ=−104025−1500⋅π180									(19)

Starting now from the positions in fig.2 as initial values, the functions expressing the inner rotors will be structured in the same way as those given by (13) – (18). The new position is shown in fig.4. This helps us checking the proposed algorithm.

Figure 4: Checking positions

3. CONCLUSIONS

By changing the values of the rotation angles, the program allows the visualization of any position of the inner rotors. The representations in fig.2 and 3 use higher values of the angles than those occurred in real life, in order to obtain a clear representation. In fact the angles are smaller and can be determined by the small transducers assembled on the inner rotors. The information are transmitted to various gyroscopic devices, which provide the pilot with everything he needs to assure the flight safety.

REFERENCES

[1] Cotoros D., Contributions to the Theory, Calculus and Construction of the Directional Gyros from their Stability Point of View. Ph. D. dissertation, Transilvania University of Braşov, 1998.

Cotoros D., Contribuţii la teoria, calculul şi construcţia aparaturii de bord girodirecţionale din punct de vedere al stabilităţii mişcării acesteia,  Teză de doctorat, Universitatea “Transilvania”, Braşov 1998.

[2]  *** MATHCAD Version 2.5., User’s Guide. MathSoft Inc., Cambridge, MA, USA.

[3] Cotoros, D., “The Utilisation of the Gyroscopic Compasses for Land Vehicles”, CONAT, vol. II, Braşov, 1996.

[4] Derek Bray, External Ballistics, course, Cranfield University, 2005