|  | 1st International Conference Computational Mechanics and Virtual Engineering  COMEC 2005 20 – 22 October 2005, Brasov, Romania |
| --- | --- |

THE STUDY OF SOME NONLINEAR DYNAMICAL SYSTEMS WITH DISTRIBUTED PARAMETERS

M. Lupu1, F. Isaia2

1 “Transilvania” University of Brasov, Brasov, ROMANIA, m.lupu@info.unitbv.ro

2 “Transilvania” University of Brasov, Brasov, ROMANIA, isaiaflorin@yahoo.com

Abstract: In this paper, the oscillations of some nonlinear dynamical systems with specially distributed parameters are studied. The nonlinear part of the equations depends on the speed under polynomial form. Such situations generalize the Van der Pol equations with one, two, three or four parameters. It is the case of the electrical transistorized circuits (Lienard), the Rayleigh equations for nonlinear vibrations in elasticity, the aero-dynamical flutter. Here, besides the elastic force and the harmonic perturbations, we have dumping forces which contain polynomial terms in speed and in this case, the energetic perturbations can generate self-oscillations. For these systems, we study the stability of solutions in the autonomous or non autonomous case, the existence of bifurcations and limit cycles by using the criteria of Hopf and Bendixon and the Liapunov function. By applying the average method or asymptotic developments with respect to the small parameter, the resonance and the self-oscillations for these nonlinear dynamical systems are studied and the trajectories are specified.

Keywords: nonlinear dynamical systems, self-oscillations, stability, bifurcation, limit cycle.

1. INTRODUCTION

In this paper we will study nonlinear equations of the type

x+2nx+ω2x=μfx,x,t+Ft,								(1)

where μ is a small parameter and f is a nonlinear function in x, x. Particularly, we will deal with equations where the damping force x has variable coefficients which contain polynominals with x, x of the type [1], [2], [3]

x+x=μax+bx+cx+εx2−1x+λx23−1x+α−βx2−γx2+Ft.			(2)

The family of equations (2) is a subset of the so called “Van der Pol” equations (VP). They represent mathematical models for mechanic vibrations, fluid oscillations [5], electrical transistorized circuits, generating lamps [10], flutter oscillations in aerodynamics [7], dampers with friction [9], astronomical phenomena, etc. The forces Ft can also be harmonic.

Particularly, equations (2) can also be quasilinear:

x+ω2x=μ1−x2x+μksinλt								(3)

or

x−α1−βx2x+x=0.									(3’)

From equations of type (3), with the change z=x and by integration, the Rayleigh equation can be obtained [4], [9]. Another form of (VP) equation is

x+x=μαx+βx3+γx5,									(4)

which represents the self-oscillating motion with dry friction of a rigid body on a driving belt with constant speed [2], [10]. A similar equation is the mathematical model for an aerodynamic flutter activated by the elastic force and the wind which implies a polynominal resistance force [7]:

x+bx=x1−a+cx3−dx5.									(4’)

A general form of the (VP) equation is [1]

x+x−xα−βx2−γx2=0, α∈R, β>γ>0.						(5)

The study of the stability of solutions is performed by considering the phase space x,y and by studying the behavior of solutions in the neighborhood of equilibrium points. In fact, we attach to every equation presented bellow, a system of the form

x=y, y=hx,y										(6)

which is a special case of the systems

x=Px,y, y=Qx,y.									(6’)

The equilibrium points are the solutions x,y of the algebraic system with unknownsx,y [4]

Px,y=0, Qx,y=0.									(7)

Because of nonlinear forces with distributed parameters, there appear perturbations of the stability, bifurcations, limit cycles, self-oscillations. The criteria and the methods used herein are well known: the first approximation, Hopf and Bendixon bifurcations [6], [10], the method of the Liapunov function [4], the Van der Pol method with the method of constants variation, asymptotic approximations (Krilov-Bogoliubov-Mitropolski) [2], [3], [10], resonances, etc.

Now, we exemplify the technique for the choice of the sufficiently small and adimensional parameter μ and the obtaining of the quasilinear equation. So, by starting from equation

mx+cx+kx =A−Bx2x−Cx3+Dsinωt,

by dividing to m, by denoting c/m=2n, k/m=p2, by performing the change  ωt=τ, x=dxdτω=x'ω, x=ω2x'' and by choosing the sufficiently small parameterμ as the coefficient of x', we obtain the notations p2−ω2/ω2=μχ,  2n/ω=μand the equation

x''+x=μ−χx+α−βx2x'−γx+f0sinτ.						(8)

For the other equations, we can similarly proceed in order to choose the parameterμ.

2. STABILITATY. BIFURCATIONS. SELF-OSCILLATIONS. RESONANCE

We will do this study for the (VP) equations (3), (5), autonomous or non-autonomous [8], [10].

We consider the reduced equation (3) and the corresponding system in the phase space x,y

x+x=μ1−x2x,										 (9)

x=y, y=−x+μ1−x2y.									(10)

The equilibrium point is Mx=0,y=0=O. The first approximation of system (10) will be

x=y, y=−x+μy.									(11)

The characteristic polynominal with the corresponding roots will be

∂P∂xx,y−r ∂P∂yx,y.			(12)

If μ<0, then M is an asymptotic stable point for the linear system (11) as for the nonlinear system (10). In fact, for μ∈(−∞,−2], M is an asymptotic stable node and for μ∈−2,0, M is an asymptotic stable focus [2], [4].

If μ>0, then M is an unstable point for the linear system (11) as for the nonlinear system (10), the trajectories leaving M when t→∞.

If μ=0, then M is a simple stable centre for the linear system (11), so we cannot specify the stability of M for the nonlinear system (10). Therefore, we will use the Liapunov and Bendixon theorems in order to study the existence of Hopf bifurcations and stable limit cycles. We will verify the existence of a periodic trajectory in the neighborhood of μ0=0. The nonexistence of such trajectory can be verified with the Bendixon theorem [1], [10]: The system x=Px,y, y=Qx,y has a periodic solution if and only if the expression ∂P/∂x+∂Q/∂y changes its sign or takes zero value. In the case of system (10), we have ∂P/∂x+∂Q/∂y=μ1−x2 which takes zero value for x=±1. So, system (10) admits no periodic solution in the half-plane x<−1, in the half-plane x>1 or in the strip −1<x<1 and every periodic solution intersects the straight lines x=±1. We also know that if this periodic solution is a stable limit cycle, then it contains the unstable point M=O. For this case we have a Hopf bifurcation [6]: If the characteristic polynominal (12) has complex roots of  type r=αμ+iωμ, with αμ0=0, αμ<0 for μ<μ0, αμ>0 for μ>μ0, ∂α∂μ|μ=μ0>0 and ωμ0=ω0≠0, then for μ>μ0 sufficiently small, the system x=Px,y, y=Qx,y admits a periodic solutions.

Here, μ0=0, αμ=μ2>0 for μ>0, ∂α∂μ|μ=0=12>0, ω0=1≠0 and hence, for μ>0 system (10) admits a periodic solution which is a limit cycle. In order to specify the stability of the limit cycle, we will use the complex form of system (6’). We have z=x+iy, z=x−iy, z=x+iy. By adding the two equations of (6’) we obtain

z=αμz+gz,z,μ,									(13)

where g contains monominals of degrees 2, 3 of z,z [6], [7]

gz,z,μ=g202z2+g11zz+g022z2+g212z2z+g122zz2+g306z3+g036z3+Oz4.		(14)

Define the first Liapunov coefficient to be the real number

l1μ0=Rec1μ0ω0, with c1μ0=i2ω0g20g11−2g112−13g022+g212.			(15)

We also know that sgnl1μ0=sgnReig20g11+ω20g21. We remind the topological Hopf criterion:

-if l1μ0<0, then the periodic solution is a stable limit cycle with supercritical Hopf bifurcation.

-if l1μ0>0, then the periodic solution is an unstable limit cycle with subcritical Hopf bifurcation.

In the case of system (10) we have the complex form

z=−iz+μz−z2+μ8−z3−z2z+zz2+z3, with g212=−μ8.				(16)

For μ>0, we have sgnl1μ0=sgnRe−ω0μ/8=−1. Therefore, the periodic solution is stable limit cycle which contains the unstable point O.

Remark. A different approach of system (10) or equation (16) is the use of polar coordinates: z=reiθ, z=re−iθ, z=reiθ+ir {θeiθ. One obtains a system of type r=ur,θ, θ=vr,θ which can be qualitatively studied and sometimes integrated . In our case, the system obtained in this manner is nonlinear and difficult.

In order to solve system (10), because of its nonlinearity, we will use the Van der Pol method and the averaging method which is in fact the first approximation from asymptotic methods [8], [10].

We start from equation x+ω2x=μfx,x, where for μ=0, we have the solution  x=acosωt+bsinωt. In the following, we will consider that a=at, b=bt are parameters with slow variation and by using the method of constants variation, we have

acosωt+bsinωt=0,									(17)

x=−aωsinωt+bωcosωt, x=−aωsinωt+bωcosωt−aω2cosωt−bω2sinωt,		(18)

Replacing x, x, x in that equation, we get

−aωsinωt+bωcosωt=μfacosωt+bsinωt,−aωsinωt+bωcosωt.			(19)

The solution of system (17), (19) with unknowns a,b is

a=−μωf⋅sinωt, b=μωf⋅cosωt.								(20)

We consider the polar coordinates

a=Acosθ, b=Asinθ									(21)

where A=At, θ=θt have a slow variation in time and we get

a=Acosθ−Aθsinθ, b=Asinθ+Aθcosθ.						(22)

Therefore, the solution x,y, with y=x, becomes

x=Acosωt−θ, y=x=−Aωsinωt−θ.							(23)

By averaging relations (20) with respect to period T=2π/ω and using (21), (22), we obtain

Acosθ−Aθsinθ=−μ2πfAcosωt−θ,−Aωsinωt−θsinωtdt.			(24)

Denoting ϕ=ωt−θ and solving (24), we have

A=−μ2πωfAcosϕ,−Aωsinϕsinϕdϕ=−μ2πωΦA.				(25)

When ΦA and ΨA are given, the differential system (25) and the initial conditions A0=A0, θ0=0 allow us to find the amplitude At and the phase difference θt. Because At and θt have a slow variation, system ΦA=0, ΨA=0 allows us to find the critical points A=A=const. It is easy to see that a2+b2=A2=x2+y/ω2 and therefore, the curves a2+b2=A2 define circles on the plane a,b which settle the boundaries of some circular annuli of stability or limit cycles. If Φ'A<0, then we have instability and ifΦ'A>0, then we have stability. Indeed, by perturbing the amplitude A=A+u in equation

dAdt=−μ2πωΦA,

we obtain

dudt=−μ2πωΦA+u≃−μ2πωΦ'Au.

Self-oscillations. They are oscillations produced by the energy transmitted to the system from sources with non-oscillating character: damping or resistance with dry friction, wind, maintained self-induction. Inside the motion equations, damping forces with variable coefficients appear and in certain moments, the characteristic equation admits real positive roots (instead of complex roots) which increase the amplitude. The phenomenon of resonance appears when we have perturbing external forces, while self-oscillations appear because of interaction between internal forces, particularly if these forces depend on speed with high exponents. For example, we have self-oscillations in equations

x−c1−c2x2x+kx=0,

x−c1−c2x2x+kx=0,

if the conditions x<c1/c2, respectively x<c1/c2, are fulfilled during a short period of time. After this period, the amplitude At satisfy At→A when t→∞, independently of the initial conditions.

Returning to (25), we remark that we have to find A=At,A0 and then we have to determine a limit stationary amplitude A, when t→∞. The motivation of this operation is that equation A=−μ2πωΦA represents the variation of the energy and of the mechanical work produced by the nonlinear terms fx,x of an equation of type x+x=fx,x. If the mechanical work fx,xdx=fx,xxdt is negative during a short period of time, then the energy decreases and the motion is amortized. If the mechanical work is positive during a short period of time, then the energy increases. These two situations do not assure an energetic balance for all periods, therefore the condition for the uniformity of self-oscillations is

ΦA=fAcosωt,−Aωsinωtsinωtdt=0.

We find  A which verifies ΦA=0 and At→A, when t→∞.

By performing this calculation for equation

x+ω2x=μ1−x2x≡fx,x

and using (25), we have

A=−μ2πω1−A2cos2ϕ−Asinϕsinϕdϕ=−μ2πωΦA,				(26)

i.e.

A=μωA2−A38, θ=0.									(27)

By integrating the obtained equations and by using the initial conditions A0=A0, θ0=0, we get

A=21−A02−4A02e−μωt, θ≡0								(28)

We have limt→∞At=A=2 (for μ>0). We remind that x=Atcosωt, y=−Atωsinωt and we remark that the straight line A=2 is an asymptotic line for the curves A=At, therefore the oscillations x=xt are tangent to the straight lines x=±2 and the regime of motion becomes stationary for oscillations (independently of the initial condition A0). We deduce from (28) that the trajectory x=xt, y=yt from the phase space is a spiral line which tend to a stable limit cycle when t→∞. This is the ellipse

x24+y24ω2=1,

for μ>0 in the neighborhood of point μ0=0. In this point the Hopf bifurcation holds.

The non-autonomous case. Consider equation

x+ω2x=μ1−x2x+μkcosλt.								(29)

By applying the method of Krilov-Bogoliubov-Mitropolski (K-B-M) [2], [3], [5] to equation (29), we seek for an asymptotic solution of type

x=Acosωt−θ+μx1+μ2x2+…+μNxN.							(30)

By replacing (30) in (29) and by making the identification with respect to μ0, μ1 (the first approximation) we get the following equations

dθdt=0, dAdt=μ8ω4A−A3,									(31)

d2x1dt2+ω2x1=A34sin3ωt−θ+kcosλt,							(32)

Using the initial conditions A0=A0, θ0=0, x10=0, x10=0, we find for equations (31) the solutions (28) and for the non-homogenous linear equation (32), we find the solution

x1=Atcosωt−μA332sin3ωt+μkω2−λ2cosλt.						(33)

Remark the term x1o=Atcosωt, which results from the homogenous linear equation, the perturbing term μA332sin3ωt and the term μkω2−λ2cosλt, which is due to the excitation. We also remark that if λ↔ω, then, besides the self-oscillations, we have resonance and hence instability. Remark that by using the linearization and asymptotic theories, the nonlinear terms fx,x can introduce in the final solution harmonic, sub-harmonic or super-harmonic terms which can make a resonance with cu the own oscillation x+ω2x. It is the case of term μA332sin3ωt. These forced appearances request separated solutions [2], [3].

We have limt→∞At=2 with self-oscillations and xt≤2+μ/4+μk/ω2−λ2, y≤M. This means that the elongation is bounded and that we have a simple stability in the point O0,0 which is a simply stable centre.

3. THE STUDY OF THE GENERALIZED VAN DER POL EQUATION

We can similarly study the general (VP) equation (8)

x+p2x=μA−Bx2−Cx2x+μPsinωt

which, by the change ωt=τ, p2−ω2/p2=μχ, becomes

x''+x=μ−χx+α−βx2−γxx'+Psinτ.

We can directly consider equation

x+x=α−βx2−γx2x+Psinωt								(34)

which, by the change ωt=τ, 1−ω2/ω2=μχ, becomes

x''+x=μ−χx+1−εx2−δxx'+f0sinτ,							(35)

where μ=α/ω, ε=β/α, δ=αγ/μ2.

We analyze equation (34) for the autonomous case. By considering the phase space for α∈R, 0<γ<β, we have

x=y≡Px,y, y=−x+αy−βx2y−γy3≡Qx,y,						(36)

with the equilibrium point O0,0. The first approximation around O of system (36) is

x=y, y=−x+αy,								(37)

with the characteristic equation r2−αr+1=0 and roots r1,2=α±α2−4/2. The stability of trajectories with respect to O is identical to that from Section 2, systems (10), (11): For α<0, we have asymptotic stability and  for α>0, we have instability.

For α0=0, we will study the existence of bifurcations and of limit cycles. The Bendixon-Liapunov theorem states: If the domain D contains the periodic curve C:x=Rcosθ, y=rsinθ with normal vector nRcosθ,rsinθ, if a trajectory C:x=xt,y=yt with speed vx,y intersects the curve Cin some pointM, then we have

-if v⋅n|M<0, then there is a stable limit cycle;

-if v⋅n|M>0, then there is an unstable limit cycle.

In our case, v⋅n|C=R2sin2θα−βR2cos2θ−γR2sin2θ (see (36)). It follows that, if α<0, then v⋅n|C<0 inside the circular annulus of radii R1=α/β, R1=α/γ and so there is a limit cycle inside this circular annulus. The same conclusions can be made by using the Liapunov function V=x2+y2/2.  We have V0,0=0, V>0 in R20,0, V=y2α−βx2−γy2 (see (36)). It follows that V>0 inside the ellipse α−βx2−γy2=0 with axes R1=α/β, R1=α/γ. Hence,  O0,0 is an unstable point and the domain bounded by the ellipse is a domain of stability for the limit cycle.

Now, we verify the existence of Hopf bifurcations for system (36) and α0=0 with the help of the complex form of this system:

z=−iz+α2z−z+z38γ−β−183γ+βzz2+183γ+βzz2+z38β−γ.			(38)

By the Hopf criterion (see (15)), we have sgnl1α0=sgn−3γ+β/8=−1 which assure the existence of a stable limit cycle in the neighborhood of α0=0, α>0. So, there is stable closed trajectory and all trajectories will be spiral lines which will tend to this curve and will leave O0,0 which will be an unstable point.

Now, we study equation (35) for the autonomous case with the Van der Pol method and the averaging method as they have been described in Section 2, relations (17)-(25). We start with x=Acosτ−θ, where A=Aτ, θ=θτ are parameters with slow variation. Performing the calculations, we get

A'=μ84A−A3ε+3δ.									(39)

The critical point is

A=4ε+3δ=4α3γ+β

(for α>0) and is obtained from (39) by writing

A'=0⇔μ84A−A3ε+3δ=0.

By solving the first equation of (39) and taking account of the initial condition A0=A0, we obtain

A=A1−Ce−μτ, cu C=1−AA02,							(40)

x=Acosτ−θ=A1−Ce−μωtcosωt−π2.							(41)

Note that

limt→∞At=A=4α3γ+β.								(42)

For α>0 and 0<γ<β the trajectories x,y from the phase space are spiral lines which tend, when t→∞, to the stable limit cycles x2+y2/ω2=A2 which is an ellipse.

The curve x=xt given by (41) has as wraps lines the straight lines x=±A. We also observe the existence of self-oscillations because, by (42), the amplitude At of xt increase to A when t→∞, independently of A0. So, the motion tend to an oscillating stable motion in interval −A,A.

The non-autonomous case of equation (35) can be treated with the (K-B-M) method as in Section 2. This is the purpose of a future paper.

4. CONCLUSIONS

This paper has a scientific and didactic character by making a study on the class of general Van der Pol equations and by presenting the techniques for the use of the existing mathematical apparatus and by emphasizing the stability, the bifurcations, the self-oscillations and the resonances.

REFERENCES

[1] Bellomo, N.; Preziosi, N.; Romano, A.: Mechanics and Dynamical Systems with Mathematics, Birkhauser, Boston, 2000.

[2] Deciu, E.; Bugaru, M.; Dragomirescu, C.: Vibratii neliniare ca aplicatii in ingineria mecanica, Ed. Acad. Romane, Bucuresti, 2000.

[3] Dinca, F.; Teodosiu, C.: Nonlinear and Random Vibrations, Ed. Acad. Romane, Bucuresti, 1973.

[4] Dragos, L.: Principiile mecanicii analitice, Ed. Tehnica, Bucuresti, 1976.

[5] Georgescu, A.: Aproximatii asimptotice, Ed. Tehnica, Bucuresti, 1989.

[6] Kuznetsov, Y.A.: Elements of Applied Bifurcation Theory, Springer Verlag, New York, 1998.

[7] Lupu, M.; Postelnicu, A.; Deaconu, A.: A Study on the Flutter Stability of Dynamic Aeroelastic Systems, Proceedings CDM 2001 Brasov, 37-46.

[8] Lupu, M.; Isaia, F.: The Study of Some Speed Regulators for Non-Autonomous Vibrating Mechanisms, Bulletin of the “Transilvania” University of Brasov, Vol. 12(47), 2005 (to appear).

[9] Teodorescu, P.P,: Sisteme mecanice, Vol. 4, Ed. Tehnica, Bucuresti, 2002.

[10] Voinea, R.; Stroie, I.:Introducere in teoria sistemelor dinamice, Ed. Acad. Romane, Bucuresti, 2000.