|  | 1st International Conference Computational Mechanics and Virtual Engineering  COMEC 2005 20 – 22 October 2005, Brasov, Romania |
| --- | --- |

THE CLASSICAL MODEL OF TWO SOLIDS COLISSION

Pandrea Nicolae1

1 University of Pitesti, Pitesti, ROMANIA

Abstract: The paper is a  models study  synthesis for the solids collision with and without friction. In these sense it writes the general theorems in the local reference system which contains the tangent plane and the common normal at the contact point and it extracts the equation for describing the collision process.

Keywords: percussion, pluckerian co-ordinates, restitution coefficient, inertances.

1. INTRODUCTION

In the precedent papers [4], [5], [8] the general collision without friction of free solids it studied and the paper [10] it studied the general collision of two solids with constraints and it is shown that introducing the collision inertance notion the final relation for percussion, energy of the losses speed and kinetic energy loss are formal identically with those known from speciality literature for the centric collision of two balls.

In the papers [8], [9] it studied the collision with friction and it is shown that by using the inertance notion the collision process is governed in fact by the non-linear differential equation system.

In the present paper it will analyse first the collision model with friction of two free solids and by particularisation will deduce the collision model without friction of two free solids.

In the final part it will show some aspects regarding the collision without friction of two solids with constraints.

2. THE COLLISION WITH FRICTION OF TWO SOLIDS BODIES

2.1. The spatial collision case

It considers two solids (fig.1) with the reference system CiXiYiZi,  i=1,2 of the principal inertia axis and the point O is the collision point and Oxzy the local system in which the Ox axis is the boundaries common normal having the relative initial normal speed v12x0 sense and the Oy axis is common tangent having the relative initial normal speed v12y0 sense.

The velocity speed before collision is defined by the kinematics torsors (vCi,ωi), i=1,2, respective by the vectors vCi,ωi projections column matrix,   i=1,2 in the reference systems axis CiXiYiZi.

| Vi0=Vix0Viy0Viz0ωix0ωiy0ωiz0T, i=1,2 | (1) |
| --- | --- |
| Vi=VixViyVizωixωiyωizT | (2) |

# Figure 1

The inertia matrices of two solid bodies, relative to reference systems CiXiYiZi, respective their inverses can be written as:

| Mi=mi000000mi000000mi000000Jix000000Jiy000000Jiz Mi−1=mi−1000000mi−1000000mi−1000000Jix−1000000Jiy−1000000Jiz−1 | (3) |
| --- | --- |

If one notes vix,viy,viz the projection’s velocity vi, i=1,2 on the axis Ox, Oy, Oz, for the point O which belong to solid body 1 respective 2 then the relative tangential velocity v12t at a given time, of the point O1 reported to  the point O2 is given by the expression:

| v12t=(v1y−v2y)j+(v1z−v2z)k | (4) |
| --- | --- |

where i,j,k are local axis versors Ox, Oy, Oz and it makes with axis Oy the angle  (fig.2) defined by the relation:

| tgα=v1z−v2zv1y−v2y | (5) |
| --- | --- |

Denoting with P the normal percussion intensity (at a given time) one results that the tangential percussion Pt which acted on the first solid body is:

Figure 2

| Pt=−μP(cosα j+sinα k) | (6) |
| --- | --- |

where  is a coefficient analogue  with that by the slip friction.

Denoting forward with aij,bij,cij, i=1,2, j=1,2,3 the directors co-sinus of the axis Ox, Oy, Oz, relative to the systems CiXiYiZi, i=1,2  and denoting with dij,eij,fij i=1,2, j=1,2,3 the projection’s vectors CiO×i,  CiO×j,  CiO×k on the axis system CiXiYiZi it can be write the matrix:

| [B]=a1ib1ic1id1ie1if1ia2ib2ic2id2ie2if2ia3ib3ic3id3ie3if3i i=1,2 | (7) |
| --- | --- |

and then from the general theorems of collision one deduces the differential equations:

| MdVi=(−1)iB 1μcosαμsinα dP i=1,2 | (8) |
| --- | --- |

dVi and dP are the differential variations of the sizes Vi, P.

It uses the form (8) to write the general theorems because the angle  is variable.

Writing the relation (8) in the following way:

| dVi=(−1)iM−1B 1μcosαμsinα dP i=1,2 | (9) |
| --- | --- |

and passing to reference system Oxyz with the aim of the position matrix,

| Ti=a1ib1ic1id1ie1if1ia2ib2ic2id2ie2if2ia3ib3ic3id3ie3if3i000a1ib1ic1i000a2ib2ic2i000a3ib3ic3i i=1,2 | (10) |
| --- | --- |

of the inertances

| gjki=ajiaki+bjibji+cjickimi+djidkiJix+ejiekiJiy+fjifkiJiz | (11) |
| --- | --- |
| hjki=ajiakiJix+bjiekiJiy+cjifkiJiz | (12) |

and the inertances matrix:

| Gi=g11ig12ig13ig21ig22ig23ig31ig32ig33ih11ih12ih13ih21ih22ih23ih31ih32ih33i | (13) |
| --- | --- |

it obtains the differential equation systems:

| dvidP=(−1)iGi1μcosαμsinα i=1,2 | (14) |
| --- | --- |

where

| vi=vixviyvizωixωiyωizT | (15) |
| --- | --- |

From the system (14) one deduces the system with relative speeds:

| v12=v1−v2 | (16) |
| --- | --- |
| dv12dP=−Gi 1μcosαμsinα | (17) |

where

| G=G1+G2 | (18) |
| --- | --- |

To the systems (14), (17) one adds the  relation (5) which can be written as:

| tgα=v12zv12y | (19) |
| --- | --- |

The differential equation systems (14), (18) can be integrated numerically with initial condition:

| P=0; v12x=v12x0; v12y=v12y0; v12z0=0; α=0; ω12x=ω12x0; ω12y=ω12y0; ω12z=ω12z0 | (20) |
| --- | --- |

Considering the integration step sufficiently small one obtains the normal and tangential relative speed (v12n,v12t) variations (fig. 3) where:

| v12t=v12y2+v12z2 | (21) |
| --- | --- |

and for the angle  depending by percussion P intensity.

| a) | b) | c) |
| --- | --- | --- |
| #### Figure 3 |  |  |

If we use the Newton model then the restitution coefficient it defines by relation:

| kN=−v12xfv12x0 | (22) |
| --- | --- |

and from the figure 3.a one obtains the compression percussion Pc and slackening percussion Pd with their aim can be calculate the Possoin restitution coefficient

| kP=PdPc | (23) |
| --- | --- |

which due to non-linear speed v12xvariation   with  P, differs by the Newton restitution coefficient.

2.2. The planar collision case

If the two solids are plates (fig.4) with motion in a plan perpendicularly by the axis CiZi, i=1,2 then =0 and from the system (17) one obtains the equations:

| dv12xdp=−(g11+μ g12); dv12ydp=−(g21+μ g22); dω12zdp=−(h31+μ h32) | (24) |
| --- | --- |

where

| gjk=gjk1+gjk2; hjk=hjk1+hjk2 | (25) |
| --- | --- |

## Figure 4

or

| g11=1m1+1m2+(f11)2J1+(f12)2J2; g12=g21=f11f21J1+f12f22J2; g22=1m1+1m2+(f21)2J1+(f22)2J2; h31=f11J1+f12J2; h32=f21J1+f22J2 | (26) |
| --- | --- |

By integration of elementary differential equations (24) one obtains the equations:

| v12x=v12x0−P(g11+μ g12);v12y=v12y0−P(g21+μ g22); ω12z=ω12z0−P(h31+μ h32) | (27) |
| --- | --- |

In the case when the tangential relative speed v12y doesn’t have the sign changing then for the Newton and Poisson models it obtains the same final results as:

| v12xf=−kNv12x0; Pf=(1+kN)v12x0g11+μ g12; v12yf=v12y0−Pf(g21+μ g22); ω12zf=ω12z0−Pf(h31+μ h32) | (28) |
| --- | --- |

If the tangential relative speed is nullified on the percussion range then denoting with P the percussion for that v12y=0:

| P=v12y0g21+μ g22 | (29) |
| --- | --- |

and denoting with v12x, ω12xthe relative speed respective the relative angular velocity which correspond to percussion P.

| v12x=v12x0−P(g11+μ g12) | (30) |
| --- | --- |
| ω12z=ω12z0−P(h31+μ h32) | (31) |

taking into account that if v12y<0 then   must changed with -, it deduces in the hypothesis g12−μ g22>0 the relations:

| v12x=v12x0−P(g11+μ g12) if P<P | (32) |
| --- | --- |
| v12y=v12y0−P(g12+μ g22) if P<P | (33) |
| ω12z=ω12z0−P(h31+μ h32) if P<P | (34) |

For:

| P>P=v12x0g11+μ g12 | (35) |
| --- | --- |

it obtains the diagrams from figure 5 and results for the Newton model the expressions:

Figure 5

| Pc=P; v12xf=−kNv12x0, Pf=P+v12x+kNv12x0g11−μ g12, Pd=Pf−Pc, v12yf=−(Pf−P)(g12−μ g22), ω12zf=ω12z−(Pf−P)(h31−μ h32) | (36) |
| --- | --- |

and for the Poisson model the expressions:

| Pc=P; Pf=Pc(1+kP); v12xf=v12x−(Pf−P)(g12−μ g22); v12yf=−(Pf−P)(g12−μ g22); ω12zf=ω12z−(Pf−P)(h31−μ h32) | (37) |
| --- | --- |

By analogy for:

P<P

it obtains the representations from figure 6 and it results for the Newton model the expressions:

# Figure 6

| v12xf=−kNv12x0; Pf=P+v12x+kNv12x0g11−μ g12;Pc=P+v12xg11−μ g12; v12yf=−(Pf−P)(g12−μ g22); ω12zf=ω12z−(Pf−P)(h31−μ h32); | (37) |
| --- | --- |

and for the Possoin model the expressions:

| Pc=P+v12xg11−μ g12; Pf=Pc(1+kP); v12xf=v12x−(Pf−P)(g12−μ g22); v12yf=−(Pf−P)(g12−μ g22); ω12zf=ω12z−(Pf−P)(h31−μ h32) | (38) |
| --- | --- |

In the case when g12−μ g22<0then for P>P it produces a “tangential paste” (v12y=0) which makes that tangential percussion to nullify that leads to equalizing with zero of the terms which contain - in the relations (32), (33), (34).

3. THE FREE RIGID SOLIDS COLLISIONS

Without friction (=0), the first row of the relation (14) write as:

| dvixdp=(−1)ig11i, i=1,2 | (39) |
| --- | --- |

where g11i is inertance which can by simply noted with giand to rely on the relation (11) write as:

| gi=1mi+di2Jix+ei2Jiy+fi2Jiz, i=1,2 | (40) |
| --- | --- |

di, ei,fi    being the CiO×i vectors couched on the local axis CiXi, CiYi, CiZi.

In the relation (39) it can replace the index x with n (normal) and then by integration it obtains the relation:

| v1n=v1n0−Pg1; v2n=v2n0+Pg2; v12n=v12n0−P(g1+g2) | (41) |
| --- | --- |

Taking into account by the restitution coefficient Newton (identical with Poisson’s in without friction case):

| k=v12nfv12n0 | (42) |
| --- | --- |

it obtains for the final percussion:

| Pf=(1+k)v12ng1+g2 | (43) |
| --- | --- |

The relation (43) is identical with that it obtains at the centric collision of two balls.

Starting from the general theorems which lead to matrices solutions:

| MiVi−Vi0=(−1)iPSi, | (44) |
| --- | --- |

where

| Si=aibicidieifiT | (45) |
| --- | --- |

ai,bi,ci are the vector i projections on the CiXi, CiYi, CiZi axis.

It deduces the equalities:

| Vi−Vi0=(−1)iPMi−1Si, i=1,2 | (46) |
| --- | --- |

The energy of the losses speed Epi, for a solid body write as:

| Epi=12ViT−Vi0TMiVi−Vi0 | (47) |
| --- | --- |

and due to relation (46) and to equality:

| gi=SiTMi−1Si | (48) |
| --- | --- |

it results

| Epi12P2gi | (49) |
| --- | --- |

For the both of solids it obtain:

| Ep=12P2(g1+g2) | (50) |
| --- | --- |

The kinetic energy loss ΔEi of a body solid write:

| ΔEi=12Vi0TMiVi0−12ViTMiVi | (51) |
| --- | --- |

and taking into account relation (49) also by the equality:

| vin=SiTVi | (52) |
| --- | --- |

it results:

| ΔEi=−Epi−(−1)iPvin | (53) |
| --- | --- |

For the both solids body the expression for the kinetics energy losing it obtains:

| ΔE=(1−k2)(v12n0)2(g1+g2) | (54) |
| --- | --- |

which is formal identical with that from the centric collision of two balls.

4. THE SOLIDS  WITH CONSTRAINT COLLISION WITHOUT FRICTION

  - The collision of solid with constraints

It considers that the body by mass m, couched in the co-ordinate system CXYZ of principal inertia axis having the principal inertia moments Jx, Jy, Jz is in collision in the point A by the percussion P with u unitary vector.

It denotes with (a,b,c), (d,e,f) the projections by the axis CX, CY, CZ of the u, CA×u vectors and one denotes with {S} the column matrices of the  pluckerian co-ordinates:

| S=abcdefT | (55) |
| --- | --- |

Denoting with Ui i=1,2,…n the pluckerian coordinates matrices which define the constraints and denoting with Qi, i=1,2,…n the percussions amplitudes from constraints, with the help of the notations (1), (2), (3) the collision general theorems lead to matrix equation:

| MV−V0=i=1nQiUi+PS | (56) |
| --- | --- |

Using the notations:

| U=U1U2…Un Q=Q1Q2…QnT | (57) |
| --- | --- |

the equation (56) becomes:

| MV−V0=UQ+PS | (59) |
| --- | --- |

The solid having n sample constraints, one results that have 6-n  degree of freedom and then the speed V represents a linear combination by 6-n  independents vectors D1,D2,…D6−n so:

| V=i=16−nξiDi | (60) |
| --- | --- |

where ξ1,ξ2,…,ξ6−n are the real kinematics parameters.

Using the notations:

| D=D1D2…D6−n | (61) |
| --- | --- |
| ξ=ξ1ξ2…ξ6−nT | (62) |

the relation (59) becomes:

| MDV−V0=UQ+PS | (63) |
| --- | --- |

It knows [14] that the velocity space is orthogonal to the constraint percussion space, so:

| DTU=0 | (64) |
| --- | --- |

and by multiplying with DT at the left part of the relation (63) and using the notation:

| Mred=DTMD | (65) |
| --- | --- |

from (63) one deduce the expression:

| ξ−ξ0=PMred−1DTS | (66) |
| --- | --- |

which multiplying at the left with [D] becomes:

| V−V0=PDMred−1DTS | (67) |
| --- | --- |

If one multiplies with ST  the relation (67) and knowing that:

| STV=vn | (68) |
| --- | --- |

one deduces the equality:

| vn−vn0=PSTDMredDTS | (69) |
| --- | --- |

Defining like “circling inertance”  the parameter g defines by the expression:

| g=STDMred−1DTS | (70) |
| --- | --- |

one obtains the expression:

| vn−vn0=Pg | (71) |
| --- | --- |

which is formal identical with that from the centric collision of two balls.

The energy of the losses speed defined by the relation (47) after successive transformation becomes:

| Ep=12P2g | (72) |
| --- | --- |

and analogically the kinetic energy loss is:

| ΔE=−Ep−Pvn | (73) |
| --- | --- |

  - The collision of two solids with constraints

At the collision of two constraints solid body, with the help of the inertances:

| gi=SiTDiMiredDiTSi, i=1,2 | (74) |
| --- | --- |

where:

| Mired=DiTMiDi | (75) |
| --- | --- |

one obtains the relations:

| v1n−v1n0=−Pg1; v2n−v2n0=Pg2 | (76) |
| --- | --- |
| v12n−v12n0=−P(g1+g2) | (77) |

and with the help of the restitution coefficient:

| k=−v12nv12n0 | (78) |
| --- | --- |

one deduces the final percussion value:

| P=(1+k)v12n0g1+g2 | (79) |
| --- | --- |

The energy of the losses speed is:

| Ep=E1p+E2p=12P2(g1+g2) | (80) |
| --- | --- |

and the kinetic energy loss is:

| ΔE=(1−k2)(v12n0)22(g1+g2) | (81) |
| --- | --- |

As application one considers the rest bar by length 2l  and inertia moment J reported to C1 in collision with a ball by mass m and velocity v0 (figure 7) for that it requires to determine the percussion P and the kinetic energy loss ΔE.

For the bar   1   in rotation movement one obtains the expressions:

D1=000001T; S1=00−100lT

and results:

M1red=DTMD=J

g1=S1DM1red−1DTS=l2g

and for the ball by mass m one obtains g2=1m.

Figure 7

It obtains the results:

P=(1+k)v01m+l2J; ΔE=(1−k2)v0221m+l2J which can be easily verified by direct calculus.

REFERENCES

- Brach R.M.: Friction, restitution and energy loss in planar collision. ASME. Journal of Applied Mechanics, vol. 51, 1984.

- Brach R. M.: Rigid body collisions. ASME. Journal Applied Mechanics, vol. 56, 1989.

- Keller J.B.: Impact with friction. ASME. Journal Applied Mechanics, vol. 93, 1986.

- Pandrea N.: Studiul general al ciocnirilor cu ajutorul vectorilor duali. Studii şi Cercetări de Mecanică Aplicată, 36, 6, 1977.

- Pandrea N.: Asupra ciocnirii solidelor. St. Cerc. Mec. Apl. Tom 36, nr. 6, 1987.

- Pandrea N., Pandrea Marina: Asupra ciocnirii cu frecare a două solide libere. Buletinul Universităţii din Piteşti, Seria Mecanică Aplicată şi Electromecanică, 2000.

- Pandrea N.: Elemente de mecanica solidelor în coordonate pluckeriene. Editura Academiei Române, Bucureşti, 2000.

- Pandrea N., Tabacu Şt.: Asupra ciocnirii cu frecare a solidului. Buletinul Universităţii din Piteşti, Seria Mecanică Aplicată, vol.2 (18), 2003.

- Pandrea N., Pandrea Marina, Stanescu N.D.: Model mathematic al ciocnirii cu frecare a solidului. Bul. Univ. Petrol-Gaze Ploieşti, vol LV, nr. 4, 2003.

[10] Pandrea N.: About collision of two solids with constraints. Rev. Roum. Sci. Techn. Mec. Appl. Tom 49, nr.1-6, 2004.

[11] Smicală I., Smicală S.C.: Consideraţii privind ciocnirea instantanee a două corpuri ţinând seama de efectul percutant al forţelor de frecare. Tenth Conf. Mech. Vibr. Tom 47, Timişoara, 2002,

[12] Smith E.C., Liu, P.P.: Coefficient of Restitution, ASME. Journal of Applied Mechanics, vol. 59, 1992.

[13] Stronge W., J.: Rigid body collisions with friction. Proc. Royal Soc. London, A.431, 1990.

[14] Voinea R., Pandrea N.: Contribuţii la o teorie matematică generală a cuplelor cinematice. IFTOMM SYROM, vol. B., 1973.