|  | 1st International Conference ²Computational Mechanics and Virtual Engineering ² COMEC 2005 20 – 22 October 2005, Brasov, Romania |
| --- | --- |

Longitudinal stability of WIG

First A. Zelenyak Laurentiu Emanuel 1, Second B. Deliu Gheorghe 1

1 “TRANSILVANIA” University, BRASOV, ROMANIA, c-mec@unitbv.ro

Abstract: For many centuries man has been travelling over the worlds seas at ever increasing speeds. New generations of ships are often faster than the ones they replace. New technologies were introduced in order to make the speed increase possible. Conventional displacement mono hulls could no longer keep up and multi hulls and planning hulls were introduced. Even higher speeds were achieved with hydrofoils and air cushion vehicles. The practical maximum speed of all marine craft mentioned so far lies around 100 km/h. The drawback of the recent trend for high speed marine craft is the increased power requirement and fuel consumption. It is very unlikely that any "conventional" marine craft will be able to operate at much higher speeds with acceptable fuel efficiency.

Keywords: ekranoplane, stability, dynamics

1. INTRODUCTION

Longitudinal stability of vehicles operating in ground effect is very critical. For aircraft this is not generally a problem, since for them it is only a transition phase. Even if an aircraft would be unstable in ground effect - which they often are - it is not a problem, because it will be either on the ground or in free air before the effects of the instability can become dangerous. For Wing In Ground-effect (WIG) craft and hydroplanes, which are operating in ground effect for extensive periods of time, longitudinal instability can be a severe problem and can lead to accidents. Almost everybody has seen race boats "flip over" on television, not many people have seen WIG craft do the same thing, but it has happened too many times unfortunately.

In the remainder the cause for this instability will be shown in general terms and the theory of static stability on ground effect will be explained. Furthermore theoretical criteria for stable WIG craft will be derived and finally these criteria will be translated to practical design terms.

Figure 1: Front view of L-325 in cruise

2. Forces, moments and their derivatives

Many parts of a WIG craft contribute to the lift and drag. Generally only the wing and tail surfaces are considered, especially in an initial aerodynamic analysis. For an analysis of the longitudinal stability of the craft it is convenient to use the centre of gravity (CG) as the origin of the coordinate system and to transform all the relevant forces and moments to this point. The resulting forces and moments are: lift (L), drag (D) and pitching moment (M). Instead of directly using them, they are made non-dimensional by dividing them by ½·r·V²·S, resulting in the lift, drag and pitching moment coefficients: CL, CD and CM.

Two variables are very important for considerations about longitudinal stability in ground effect: the height above the surface (H) and the pitch angle (a). Height is made dimensionless with the chord length c, so h = H/c. Note that instead of angle of attack, the term pitch angle is used [1].

Very important parameters in stability analysis of WIG craft are the derivatives of CL and CM with respect to pitch angle and non-dimensional height h.

The above mentioned derivatives are written in the following way for convenience:

CLa = dCL / da

CMa = dCM / da

CLh = dCL / dh

CMh = dCM / dh

3. Aerodynamic centers

Apart from the centre of gravity, two more centers can be recognized in a WIG craft. These are defined points, not physical positions like CG, but they will turn out to be very important in stability considerations. These centers are called aerodynamic centers. An aircraft only has one, but a WIG craft has two.

The aerodynamic centre, also known in aircraft, is the point where CM remains constant with changing pitch angle. It is denoted (aerodynamic) centre in pitch here. Its definition is:

Xa = CMa / CLa

The (aerodynamic) centre in height is the point where CM remains constant with changing height. It is defined as:

Xh = CMh / CLh

Both centers are expressed in non-dimensional x-coordinates. The positive direction of the x-axis is upstream and the values represent a percentage of the wing chord.

4. Longitudinal stability

For a WIG craft in stationary cruise condition there must be equilibrium of forces and moments. In practice this means that the CG must lie on the working line of the resulting vector of the aerodynamic forces. So CM must be 0 in the cg. In order to remain in this stationary condition any disturbance must be counteracted with a force or moment in the opposite direction, so that the craft returns to its original condition. This principle is called (static) stability. 
Even if the counteracting forces occur a craft can still be unstable in the case that the forces are not sufficient or too big. In this case the craft is statically stable, but dynamically unstable. In the remainder only static stability will be covered in-depth.

Experience has shown that when a WIG craft is statically stable, it will also be dynamically stable, except for very small ground clearances where the static stability is too high, leading to dynamic instability. This area is very narrow however and is only a transitional phase during start and landing.

Two different movements in the longitudinal direction can be recognized: pitch and height. For an aircraft height stability is irrelevant. A WIG craft must be stable in both height and pitch. So after a disturbance not only the pitch angle of the WIG craft, but also the height above the surface must be restored to their initial values.

5. Static pitch stability

Even though a WIG craft is not an aircraft the standard criterion for static pitch stability of aircraft applies to WIG craft too and must be satisfied:

CMa < 0

The validity of this expression is easy to see: an increased pitch angle should be counteracted with a negative pitching moment. Note that a nose-up pitching moment is positive.

6. Static height stability

Analogous to the expression for static pitch stability, one might expect that the condition for static height stability is:

CLh < 0

In this case an increased height would indeed lead to a decreased lift and apparently a stable condition, but the fact that the moment coefficient also changes due to the height change is not taken into account. So the above expression is only valid when considering a trimmed condition, in other words: CM must be held constant (=0). Taking this into account the actual condition for static height stability as found by Staufenbiel is:

CLh - CLa ·CMh / CMa < 0

Irodov came to a different expression of the static height stability criterion, which can however be shown to be the same as Staufenbiels. This criterion uses the aerodynamic centers rather than the derivatives. This leads to the very simple equation:

Xa - Xh < 0

Note that x is positive in the upstream direction [2].

Figure 2: The influence of ground effect on

induced drag according to Wieselsberger

The latter criterion will be used for further explanation, since it is a more practical way to approach the stability problem. The criterion for static height stability is that the centre in height is located upstream of the centre in pitch. 
The value of Xa - Xh is often referred to as the stability margin. Russian experts concluded on the basis of their experience that this stability margin should in cruise have a value of approximately -0.1. Although any negative value will give static stability, a value of about -0.1 will ensure a good combination between (static and dynamic) stability characteristics and maneuverability. A lower value will give the craft a very marginal stability, it will take a while for the craft to restore to its original position. A higher value may lead to (unstable) oscillations.

7. Design parameters that affect static height stability

Generally a wing will be unstable in height. There are some parameters which strongly affect height stability and overall behavior of the WIG craft. The most known solution to stabilize a WIG craft is to give it an enormous horizontal tail. Some of the early Ekranoplans had a horizontal tail with more than a third of the wing area. Lippisch showed that the platform of the wing can positively influence stability, thus reducing the required tail volume. In the later Ekranoplan designs special ground effect airfoils were introduced that also enhanced stability. Finally stability is also influenced by the position of the CG.

7.1. Horizontal tail

Although a wing can be made stable in ground effect without a horizontal tail (in contrast to what some sources claim), this wing will only be stable in relatively small ground clearances. A horizontal tail increases height stability by shifting Xa rearward. Usually, for an isolated wing, Xa is located upstream of Xh. Addition of a horizontal tail can actually move Xa from this position to a position rearward of Xh, thus stabilizing the WIG craft. 
Although in theory a horizontal tail does not have an influence on the position of Xh, in practice this is not true due to the influence of the wing tip vortices on the horizontal tail. This shift of Xh partially cancels out the positive effect of the Xa shift, which makes a horizontal tail somewhat less efficient than expected.

7.2. Airfoil and platform

The choice of airfoil and wing platform have a tremendous effect on the stability margin. The so-called Lippisch platform has proven to be more stable than a square wing. The Lippisch platform is a forward swept wing with strong taper and negative dihedral, sometimes also called a reversed delta. This name can be confusing while the leading edge may even be swept forward.

The influence of the airfoil section may even have a greater impact on stability than the platform of the wing. Unfortunately this is an area which is relatively unexplored. For "standard" wings there are several good sources of airfoils with a lot of theoretical and experimental data, NACA airfoils may be the most popular example. The only airfoil family so far that has been specifically designed for ground effect is the DHMTU family and even then there is no large collection of data [3].

Some general remarks may be helpful in selecting the right airfoils for a WIG craft:

- Due to the low aspect ratio of most WIG craft, the flow has a very strong three dimensional character, so designing on the basis of two dimensional airfoil data is generally not very useful.

- Hardly ever will one wing section give good results along the entire span. Adaptation of the foil shape on the local requirements is necessary and will lead to much better overall performance.

- A general rule to increase longitudinal stability of the wing is to unload the rear and load the front. This is also the thought behind the S-shaped or deflexed airfoils. These foils have an S-shaped mean line, concave near the leading edge and convex further aft (as seen from below). The disadvantage of these foils is that CLmax is often fairly low.

8. Conclusion

The above information is interesting from an academic point of view, but in order for it to have any practical value it must be possible to calculate or estimate the locations of the aerodynamic centers. This can be done either by CFD (e.g. Auto wing or with wind-tunnel tests. In both cases a matrix of CL and CM values must be generated at different heights and pitch angles.

The most appropriate design method will be to use CFD methods in the early stage of development, where a lot of parameters can be changed without much effort and once the configuration is frozen some wind tunnel tests may be run to validate the CFD work and to fine-tune the configuration.

REFERENCE

[1] Edwin P.E. van Opstal: S.E. Technology, 1998

[2] Fishwick, S.: Low flying boats, 2001

[3] P.A. Wilson, A. Nebylov: Ekranoplanes - Controlled Flight Close to the Sea, 2001