|  | 1st International Conference Computational Mechanics and Virtual Engineering  COMEC 2005 20 – 22 October 2005, Brasov, Romania |
| --- | --- |

ESTIMATION OF THE INFLUENCE OF MICRO-PROFILE OF ROLLING TRACK ON VEHICLE DYNAMICS

Mihai I. Toader1, Nicolae Herişanu2

1 Politechnic University of Timişoara, Romania, toader@mec.utt.ro

2 Politechnic University of Timişoara, herisanu@mec.utt.ro

Abstract: The paper studies the motion of a vehicle on a rolling track having a microprofile that influences its behaviour. It is considered that the microprofile is a random stationary process, with an index of regularity rapidly decreasing; the spectral composition is obtained by a linear transformation. It is solved an isoperimetrical problem that gives the characteristic of transfer of the microprofile. The behaviour of a dynamic system with three degrees of freedom is analyzed, in the conditions of the determined microprofile.

Keywords: micro-profile, random process, transfer characteristic, spectral density

1. INTRODUCTION

Taking into account the micro-profile of the rolling track represents in fact a valorization of an important excitation source appearing in the vehicle rolling. In the most of already studied situations it was considered the rolling track having a sinusoidal shape. The vehicle is assimilated with a material point tied of an elastic element, without mass, whose bottom is rolling along the rolling track.

The profile of the rolling track is build of three distinct components: the macro-profile, the micro-profile and the ruggedness, which influence in a different way the vehicle behavior.

The micro-profile is considered to be shaped by long and gentle dislevelments between 10 centimeters to 100 meters. This one generates vibrations of the vehicle structure, but because of its small wave-lengths, they do not generate major modification on the working regimes of the engine.

The use of the micro-profile as an excitation source has some specific features. First of all, the amplitude of the micro-profile is limited to 1 m. Then, it does not contain variable components. It is a stationary random phenomenon with the index of regularity rapidly decreasing. Thus, two arbitrary portions of the micro-profile situated to a very close distance can be considered independent, meanwhile like a profile it is a random phenomenon with a stationary increasing.

Let us consider the micro-profile h(l) whose spectral composition is obtained by the linear transform Hq, defined by:

q(l)=Hqh(l)										(1)

Where:

Hq(iλ)=0    for      λ<λm1     for      λm<λ<λM	,	λm=2πLM,   LM=2πLm				(2)

where LM and Lm are the maximum and minimum ware-lengths of the micro-profile.

The transform Hq is named the transform of micro-profile [1], and so, the micro-profile is determined by the following elements: the minimum parameters of vibrations of the vehicle; the limiting value of the amplitude of micro-profile; the limiting value of the maximum frequency.

For the given micro-profile h(l), it is considered that Δh is the deformation of the elastic elements, corresponding to the vehicle, during its displacement. If it is noted by A the transform corresponding to the vehicle, then, like in (1), it results that Δh=Ah. If Δq is the deformation of the elastic elements of the vehicle, in its motion on the micro-profile q, it results Δq=Aq.

For the determination of the transform of micro-profile in the field of low frequencies, it is necessary that:

q≤qM,     Δh−Δq=min								(3)

The variational problem (2) is studied considering the micro-profile h(l) as a normal random generalized stationary process, and the transforms A and Hq, as linear transforms.

The conditions that determine the form of the transform Hq become:

Dq≡Mq2≤Dqmax   si   MΔh−Δq2=min						(4)

where Dqmax= (0,05-0,1)q2max [1] and M is the operator of mathematical expectation [2].

By introducing the Lagrange multiplier, the variational problem consist in the minimization of the functional, defined as follows:

f=MΔh−Δq2+λq2=min								(5)

where λ is the Lagrange multiplier.

2. THE STUDY OF THE ISOPERIMETRIC PROBLEM

In order to solve the problem (5), it is supposed that the aspect of the transform and the spectral density of the excitation proceed from the micro-profile h(l), which in the field of low frequencies is determined from the relation:

Sh(ω)=Knvan−1ω−n										(6)

where ω is the pulsation of the random process h(t), Kn-a coefficient depending on the micro-profile, and va-the speed of the vehicle, n[2,4].

The limit cases in (6) are:

Sh(ω)=K2vaω−2 										(7)

and

Sh(ω)=K4va3ω−4 										(7’)

Because the transform Hq is considered only for the field of low frequencies that are characteristic for the micro-profile of rolling track, it is sufficient that the transform A of the vehicle to have the same characteristic and, after calculus, it becomes:

A(iω)=(iω)2(iω)2+2ψ0ω0iω+ω02								(8)

where y0 is the coefficient of relative damping of the vibrations of carriage body, and ω0 is its minimum proper pulsation.

The variation of the functional:

f=12π−∞+∞A2+AHq2−A22ReHq+λHq2Shdω						(9)

in relation to Hq, leads to a functional equation that determines the characteristic of transfer of the transform Hq, under the form:

HqA2+λSh−A2Sh=F									(10)

where F is an analytic function, defined in the right half-plane [7].

When, for a certain micro-profile, there are not imposed conditions, physically feasible, from (10) it results:

HqA2+λSh−A2Sh=0 									(11)

whence    Hq=A2A2+λ.

For the particular case from (7), taking account of (8) and (10), it results:

Hq=1+λ(iω)4+λ(2ω02−4ψ02)(iω)2+λω4(iω)2+2ψ0ω0iω+ω02(iω)2−2ψ0ω0iω+ω02⋅K2(iω+0)−iω+0−−(iω+0)2−iω+02(iω)2+2ψ0ω0iω+ω02(iω)2−2ψ0ω0iω+ω02⋅K2(iω+0)−iω+0=F			(12)

where, from [7],

iω+0−1=limα→0iω−α−1=1iω−πδω

12π−∞+∞iω+0−1eiωtdω=1     pentru    t>0

12π−∞+∞iω−1eiωtdω=0,5     pentru    t>0

Knowing that the Lagrange multiplier λ<<1, it results:

1+λiω4+λ2ω02−4ψ0ω02iω2+λω04≃iω2+24λω0iω+λω02iω2−24λω0iω+λω02

and from (12), it results the simplified form:

Hq≃iω2iω2+24λω0iω+λω02								(12’)

If instead of minimizing the deformation of the springs of carriage body, it is minimized the difference of vertical accelerations of the carriage body on the micro-profile of rolling track, it obtains the transform A of form:

A(iω)=2ψ0ω0iω+ω02iω2iω2+2ψ0ω0iω+ω02

i.e. the vibrations of vehicle on micro-profile are very little different.

The dispersion of q(t) is determined with the relation [7]

Dq=12π−∞∞Hq2Shdω									(13)

which, for the forms (7) and (7’), becomes: Dq=K2va224λω0     and    Dq=K4va224λ3ω03

In the situation of a characteristic of transfer of form:

Hq=iω2iω2+2ψ1ω1iω+ω12

The expressions for the two particular cases become:

Dq=K2va212ψ1ω1=K2212ψ1θ1                              Dq=K4va3212ψ1ω13=K4212ψ1θ13						(14)

where θ1=ω1va is estimated from the condition of limitation of Dq,

θ1=ω1va≥K22⋅2ψ1Dqmax              θ1=v1va≥3K42⋅2ψ1Dqmax								(15)

where ψ10.5, and Dqmax is the maximum admitted value of the micro-profile.

From the effected analysis, it results that, when the ratio: ω1ω0=0.3−0.2, i.e. the frequency of the section of transform of micro-profile Hq is of 3-5 times smaller than the frequency of proper vibrations of the carriage body, the vibrations of the vehicle on the profile and micro-profile have important variations.

3. MODEL FOR THE STUDY

One of the cases, more frequently encountered, is the one of a dynamic system, like in the fig.1. It is considered that the frictions are negligible and that the additional mass mM is much smaller than the tyre mass, mP.

z1

z2

z3

m1

m2

m3

h

k1

k2

k3

c2

c3

The differential equations of the motion are:

Fig.1

m2z2+2c2(z2−z1)+k2(z2−z1)=0m1z1−2c2(z2−z1)−k2(z2−z1)+k1(z1−h)=0m3z3+2c3(z3−z1)−k3(z3−z1)=0						(16)

where m2 is the mass of the load, z2, z1 and z3-the vertical coordinates of the load, wheels and supplementary load, k2, k1, k3-the rigidities of the fixing springs and tyres, and c2, c3-the coefficients of the dampers of the model.

This system of linear differential equations can be transformed in an equivalent system, by introducing the following notations:

Δ=z2−z1;   Δi=z3−z1;  dz2dt=−2Ψ2ω2Δ+x1;   dx1dt=−ω22Δ;  dz0dt=2αΨ2ω2Δ+x2;   dx2dt=ω12(h−z1)+αω22Δ;dz3dt=−2Ψ3ω3Δi+x3;dx3dt=−2ω32Δi	(17)

The output signal Δi determines the characteristic of transfer of the dynamic transformer of microprofile, which has the form:

Hi(p)=mrp2+2c2p+k2c1m2m1p4+2c2(m2+m1)p3+(m2k2+m1k2+m2k1)p2+2c2c1p+k2k1⋅m3p2m3+2c3p+k3		(18)

or

Hi(p)=p2+2ψ2ω2p+ω22ω12p4+2ψ2(ω2+αω2)p3+(αω22+ω22+ω12)p+2ψ2ω2p+ω22ω12⋅p2p2+2ψ3ω3p+ω32

This can be transformed in the product of functions of transfer of three vibrating elements, of form:

Hi(p)=p2+2ψ2ω2p+ω22p2+2ψ3ω2p+ω32⋅ω12p2+2ψ2ω2p+ω22⋅p2p2+2ψ3ω3p+ω32 			(19)

This relation characterizes the field of low frequencies, as well as the one of high frequencies.

So, the problem was again reduced to an isoperimetric problem, like in section 1, i.e.

Mq−qi2=min

ACKNOWLEDGMENT

This work was supported by the Romanian Academy through the project GAR no.88/2005.

REFERENCES

[1] Pevzener, Ja.M-Tihonov, A.A.: Isledovanija statistiĉeskih svojstv microprofilja osnovnyh tipov vtomobil’nyh dorod. Avtomobil’naja promyšlennost’, nr.1, 1964

[2] Silaev A.A.: Spektralnaja teorija podressopovanija transportnyh mašin. Mašinostroenie, 1972

[3] Silov G.: Obobščenie funkij I dejstvija nad nimi. Fizmatgiz, 1959.

[4] Pandrea N., Pârlac, S., Popa D.: Modele pentru studiul vibraţiilor automobilelor, Ed.TPARG-Piteşti, 2001

[5] Harris C., Crede, C.: Şocuri şi vibraţii, Ed.Tehnică, 1968.

[6] Radeş M.: Metode dinamice pentru identificarea sistemelor mecanice, Ed. Academiei, Bucureşti, 1979

[7] Muntean M.: Introducere în dinamica oscilaţiilor solidului rigid şi a sistemelor de solide rigide. Ed.Clusium, 1997.

[8] Chiu-Shang Liu, Huei Peng: Road Friction Coefficient Estimation for Vehicle Path Prediction, Vehicle System Dynamics, vol.25, 1996

[9] Messner W., Horowitz R.: Identification of a Nonlinear Function in a Dynamic System, ASME Journal of Dynamic Systems, Measurement and Control, vol.115, 1993

[10] Lei Zuo, S.A.Nayfeh: Structured Hq optimization of Vehicle Suspensions Based on Multi-Wheel Models, Vehicle System Dynamics, vol.40, no.5, 2003