ON THE CORIOLIS EFFECTS ON THE MOTION

OF THE ELASTIC MULTIBODY SYSTEMS

Sorin Vlase, Stanciu Anca, Violeta Guiman , Niculina Nan, Marian Vasii

„Transilvania” University of Brasov, Romania

Abstract: In the study of a multi-bodies system, the elasticity of the components can be large enough so that the dynamic response can be not only quantitative, but also qualitative, different. For this reason, in some applications, particularly in the field of robotics and high-speed vehicles, is necessary to consider the elasticity of the elements and to use correspondent models. To study such systems is necessary to use numerical methods and the finite element methods (FEM) remains one of the most important tools. In the paper are established the motion equations for a general multi-bodies system with elastic elements being in a three-dimensional motion and are analyzed the influence of the Coriolis terms.

Key words: Coriolis Forces, Multibody Systems, Motion Equations.

# DINAMICS OF THE MULTIBODY SISTEMS

In the following will be established the motion equations for an elastic finite element with a general motion together with an element of the system. The type of the shape function is determined by the type of the finite element. We will consider that the small deformations will not affect the general, rigid motion of the system.

The continuous displacement field f(x,y,z,t) is approximated, in FEM, by:

f=N(x,y,z)δe(t)

where the elements of matrix [N] (the shape functions), are determined by the type of the finite element used. We consider that, for the all elements of the system, the field of the velocities and of the accelerations are known. We refer the finite element to the local coordinate system Oxyz, mobile, and having a general motion with the part of system considered. We note with  vo(Xo,Yo,Zo) the velocity and with ao(Xo,Yo,Zo) the acceleration of the origin of the local coordinate system. The motion of the whole system is refer to the general coordinate system O’XYZ. By  [ R ] is denoted the rotation matrix. If we apply the Lagrange’s equations after some algebraic operations we obtain the motion equations for a single finite element under the compact form[1],[4]:

meδe+2ceδe+ke+keE+keΩ2δe=

=qe+qe−qeiE−qeiΩ2−moeiRTro

where  Ω represent the angular velocity and E the angular acceleration with the components in the local coordinate system.

These motion equations are referred to the local coordinate system and the nodal displacement vector  δe and the nodal force vector qe+qe are express in the same coordinate system. The motion equations are true for the instantaneous position of the system. We consider that the system is „frozen” for the moment considered.

- LIAISON FORCES ELIMINATING

The unknowns in the elasto-dynamic analysis of a mechanical system with liaisons are the nodal displacements and the liaison forces. Generally, the relations between the first order derivatives of the nodal displacements can be expressed by the linear formulas:

Δ=Aq

where by Δwe have noted the nodal displacement vector and by q the nodal independent displacements. By differentiation (14)  we obtain:

Δ=Aq+Aq     	                         		(10)

The transformation relations between the displacements expressed in the global fix coordinate system Δeand the displacements expressed in the local mobile coordinate system δe are:

Δe=Reδe    				     	(11)

where index e denote the e-th element.

For a single finite element that belong to an elastic component of the system that has a general three-dimensional rigid motion with the angular velocity ω  and the angular acceleration ε (or Ω and E in the mobile co-ordinate system) we consider the motion equations obtained by the relation  (11). For the other cases the procedures are the same.

The equations are expressed in the local mobile reference system. If we write these equations in the global fix coordinate system, they keep there form:

MeΔe+2CeΔe+Ke+Keε+Keω2Δe=

=Qe+Qe−QieE−QieΩ2−MioeRTro   		(12)

We will note in the following:

Qeinertia=−QieE−QieΩ2−MioeRTro

and we can obtain finally the motion equations for the whole structure, referred to the global coordinate system, under the form:

MΔ+2CΔ+K+Kε+Kω2Δ=Qext+Qext+Qleg+Qinertie    	(13)

If we take into account the relations (18) and (20) we can write:

M (Aq+Aq) +2CAq+ K+Kε+Kω2 Aq=

=Qext+Qext+Qleg+Qinertia             	     		(14)

It can be shown that the work of the liaison forces for system can be written:

dL=  ΔTQlegdt=qTATQlegdt      		           (15)

But the work due to the liaison forces is null for an ideal system and the independence of the nodal coordinates q offer the relation:

ATQleg  = 0              	                       			(16)

that is the basic relation in the following.

The liaison between finite elements is realized by the nodes where the displacements can be equal or can be other type of functional relations between these. When two finite elements belong to two different elements (bodies) the liaison realized by node can imply relations more complicated between nodal displacement and their derivatives.

The system of differential equations obtained after the assembling procedures is nonlinear, the matrix of the left term depending on the configuration of the multi-body system. These equations can be writing under the form:

mq+cq+(k+kε+kω2)q=f

where m, k and kω2 are symmetric and c ,  kε are skew-symmetric.

- THE INFLUENCE OF THE CORIOLIS TERMS

The matrix c is skew-symmetric.  If we want to obtain the energy balance by integration, we obtain that the variation of energy due to the term skew-symmetric is null. Consequently,  the Coriolis term only transfer the energy between the independent coordinates of the system and had no role in the dissipation of the energy.

If we consider now a motion mode on the form:

q=Asin(ωt+ϕ)

and we introduce in the motion equations, where the forces are considered null, we obtain:

−ω2mAsin(ωt+ϕ)+ωcAcos(ωt+ϕ)+(k+kε+kω2)Asin(ωt+ϕ)=0

If we pre-multiply with AT we obtain:

−ω2ATmAsin(ωt+ϕ)+ATkA+ATkω2Asin(ωt+ϕ)=0

We have considered that:

ωATcA=0     and    ATkεA=0

because [c] and  kε are skew-symmetric. It results:

ω2=ATkA+ATkω2AATmA

This relation can not express, in a direct way,  the influence of the matrix [c]  in the eigen-values calculus, but this influence is present by the eigenvectors A.The term c has an influence on the values of the eigen-values. Some of the eigen-values increase and the other decrease. This variation is presented, extended, in the paper. Between these values there exist some interesting relations.

REFERENCES

[1]. Vlase, S. Elastodynamische Analyse der Mechanischen Systeme durch die Methode der Finiten Elemente. Bul. Univ. Brasov,  p.1-6, 1985.

[2]. Vlase, S. A Method of Eliminating Lagrangean Multipliers from the Equations of Motion of Interconnected Mechanical Systems. Journal of Applied Mechanics, ASME trans., vol.54, nr.1, 1987.

[3]. Vlase, S. Modeling of Multibody Systems with Elastic Elements. Zwischenbericht. ZB-86, Technische Universität, Sttutgart., 1994.

[4]. Vlase, S. Finite Element Analysis of the Planar Mechanisms: Numerical Aspects. Applied Mechanics - 4. Elsevier, 90-100, 1992.